LOCAL-FIRST SMART HOME INTEGRATION

Business, Regulatory and Technical Plan for Ontario and Quebec

Lighting  ·  Receptacles  ·  Fans  ·  Thermostats  ·  Cameras

No cloud dependency  ·  No mandatory subscription  ·  Customer-owned infrastructure

Version 1.0  ·  18 August 2026

Excludes fire, gas, carbon monoxide and all life-safety systems

# Contents

# 1. Executive Summary

This plan describes a residential and light-commercial smart home integration business serving Ontario and Quebec, built on a single differentiating principle: the customer owns the server, the data never leaves the property, and there is no mandatory subscription to any third party.

The system automates lighting, receptacles, fans, thermostats and cameras. It explicitly excludes fire detection, gas detection, carbon monoxide detection, medical alert and every other life-safety system. Intrusion detection is offered only as owner-notified local alerting with optional audible sirens. There is no central monitoring station, no police dispatch service and no alarm-receiving contract.

## 1.1 The strategic thesis

- The mass market is served; the sovereignty market is not. Canadian households buy smart devices in volume, but the professionally integrated segment is small, and every institutional competitor in it (TELUS, Bell, Vivint, Hilo) is built on a cloud-tenancy model where the provider owns the platform, the app and the recordings. A locally hosted alternative cannot be copied by these firms without destroying their own recurring-revenue architecture.

- The buyer for this proposition is geographically concentrated. The customer who pays a premium for data sovereignty is technically literate, subscription-averse and privacy-sensitive. In the National Capital Region this profile is unusually dense because of the federal public service, the defence and security supply chain, and the local technology sector. Ottawa and Gatineau are therefore the primary launch market, not merely a convenient one.

- The regulatory barrier is the moat. Quebec requires a construction contractor licence, mandatory membership in the electrical contractors corporation, workforce competency cards, and a private security agency licence for security system installation. This is expensive and slow to cross, which is exactly why so few operators have crossed it. Once cleared, the barrier works in the operator's favour.

- Recurring revenue, not hardware margin, is the enterprise value. Hardware margin compresses every year against national retail. Support, maintenance, version management and network stewardship do not. The plan makes the first year of support mandatory and included, and renewable thereafter.

- Every component must be procurable inside Canada. Certification under the Canadian Electrical Code, warranty and return handling, model continuity and cross-border cost volatility all point the same way. This constraint eliminates a large part of the hobbyist device catalogue and forces a professional bill of materials, which is a competitive advantage rather than a limitation.

## 1.2 Scope of the offer at a glance

| Domain | In scope | Out of scope |
| --- | --- | --- |
| Lighting | Switches, dimmers, relays, fixtures, lamps, scenes, schedules, occupancy control | Emergency and exit lighting, egress lighting |
| Receptacles | Controlled outlets, plug-in modules, energy metering | Any receptacle serving life-safety equipment |
| Fans | Ceiling fans on fan-rated controls, bathroom exhaust fans, timers and humidity control | Mechanical ventilation interlocks required by code, HRV/ERV core control where code-mandated |
| Climate | Line-voltage and 24 V thermostats, zone scheduling, setback automation, remote setpoint | Combustion appliance safety controls, boiler and furnace internal controls |
| Cameras | IP cameras, local NVR, AI object detection, local retention, remote viewing | Central station video verification, guard dispatch |
| Sensors and alerting | Motion, door and window contacts, water leak, temperature, humidity, air quality, local sirens, owner notification | Fire, smoke, CO, gas, medical alert, monitored intrusion alarm |
| Network | Segmented LAN, PoE, VLANs, firewall policy, encrypted remote access | Customer internet service provision |

## 1.3 What the customer gets, in their own words

The proposition, stated the way it should appear in sales material:

- Your cameras record to a computer that sits in your house. Nobody else can watch them. Not us, not a monitoring company, not a manufacturer.

- You control everything from your phone, from anywhere in the world, over an encrypted private tunnel. No ports are opened on your router and no video passes through anyone else's server.

- There is no monthly fee you are forced to pay. If you stop paying us, the system keeps working exactly as it did the day before.

- If the controller fails, the lights still work from the wall switch. Automation is added on top of a functioning house, not in place of one.

- We cannot get into your system without you turning on a switch that lets us in, and that switch turns itself off after two hours.

- You get a written document describing everything installed, every password location, every network setting and every automation, so any competent technician can take over from us.

## 1.4 Financial shape

| Metric | Target at steady state |
| --- | --- |
| Average project value, mid tier | CAD 6,000 to 12,000 |
| Gross margin, hardware | 25 to 35 percent |
| Gross margin, labour and programming | 55 to 70 percent |
| Mandatory first-year support | Included in project price |
| Renewal support plan | CAD 30 to 65 per month |
| Target renewal rate by year three | Above 70 percent |
| Projects per technician per year | 18 to 26 at mid tier |

All figures, programme details, licence requirements and tariff conditions in this plan are stated to the best available knowledge and must be confirmed against the primary source before being used in a contract, a quotation or a financing application. Items requiring confirmation are marked (V) throughout and consolidated in Chapter 14.

# 2. Scope Definition, Exclusions and Liability Boundary

Scope discipline is a commercial asset in this business. Life-safety systems in Canada are governed by separate standards, require separately certified technicians, and carry a category of legal exposure that a small integrator cannot absorb. Declaring the boundary clearly protects the company and, correctly presented, increases customer confidence rather than reducing it.

## 2.1 Mandatory scope statement

The following must appear in every proposal and every signed contract, in English or French according to the customer and the province:

- The system supplied is not a fire detection system, a fire alarm system, a smoke detection system, a carbon monoxide detection system, a gas detection system, or a medical alert system.

- The system supplied is not a monitored intrusion alarm system. No signal is transmitted to any monitoring centre. No emergency service is dispatched by the system. All alerting is delivered to the property owner and to contacts designated by the owner.

- The system does not interact with, modify, control, or depend on any life-safety system present in the property.

- Hardwired and interconnected smoke and carbon monoxide alarms required by code remain untouched and on their original circuits. No relay, dimmer, module or control device is installed on a circuit shared with interconnected detectors.

- The software supplied is open-source software licensed by third parties and distributed without warranty of any kind. The Company warrants its own installation workmanship and configuration, not the underlying software.

- The customer is responsible for responding to alerts generated by the system and for contacting emergency services where appropriate.

## 2.2 Why owner notification is the correct default

Removing the central monitoring station from the design is not a compromise. It is the better engineering and commercial decision for this market, for four reasons.

- Police in the principal target markets do not dispatch on unverified alarms. Ottawa operates a verified response policy. Montreal, Gatineau and Toronto operate alarm registration and false-alarm penalty by-laws. A monitoring contract therefore does not buy what most customers assume it buys. (V) confirm the current policy of each municipal police service in the service territory.

- It removes an entire regulatory and commercial layer. No alarm receiving centre agreement, no ULC-listed installer requirement, no signal transmission certification, no shared liability for dispatch failure.

- It preserves the sovereignty claim without asterisks. With no monitoring channel, the honest answer to "what leaves my house" is "nothing except the notification to your own phone", which is a far stronger sales position than "only event codes".

- Video-based verification delivered directly to the owner is faster and richer than a third party reading zone codes. The owner sees the actual clip on their phone in seconds and decides. This is a genuine product advantage over a legacy panel.

## 2.3 Local audible alerting

Where a customer wants deterrence, local sirens and strobes are offered as an option. Design rules:

- Sirens are driven from the automation system, not from a certified alarm panel, and are documented as a deterrent device rather than an alarm system.

- Sound level, placement and automatic cut-off must comply with the municipal noise by-law. Many Canadian municipalities require an audible alarm to silence automatically after a fixed period, commonly in the range of five to twenty minutes. (V) the by-law for Ottawa, Gatineau, Montreal and any other municipality served.

- Sirens used for intrusion deterrence must be visually and audibly distinguishable from any fire alarm sounder in the building, and must never use a temporal-three pattern, which is reserved for fire evacuation signalling.

- Every siren circuit is documented with an accessible manual disconnect.

## 2.4 Liability boundary and insurance

| Exposure | Control |
| --- | --- |
| Electrical workmanship | Licensed contractor performs or supervises all line-voltage work; permits and inspection on record |
| Property damage during installation | Commercial general liability policy, minimum CAD 2,000,000 (V) with broker |
| Design or configuration error | Errors and omissions policy; written design sign-off by customer before installation |
| Customer data exposure | Documented access policy, time-limited support access, no standing credentials, breach response procedure |
| Failure to alert | Contractual statement that the system is not a monitored alarm and carries no response guarantee |
| Open-source software defect | Contractual pass-through of upstream no-warranty terms; Company warrants configuration only |
| Worker injury | CNESST coverage in Quebec, WSIB coverage in Ontario |

# 3. Market Analysis

## 3.1 Household base and geography

Ontario and Quebec together hold roughly sixty percent of the Canadian population and just under ten million private dwellings. The addressable base is not the population but the owner-occupied single-family and semi-detached stock, since condominium and rental units restrict both wiring work and camera placement.

| Province | Population (approx.) | Households (approx.) | Principal metros |
| --- | --- | --- | --- |
| Ontario | 15.5 to 16.0 million | 5.8 to 6.0 million | Greater Toronto Area (approx. 7.0M), Ottawa CMA (approx. 1.5M), Hamilton, Kitchener-Waterloo, London |
| Quebec | 9.0 to 9.2 million | 3.7 to 3.9 million | Montreal CMA (approx. 4.4M), Quebec City (approx. 0.85M), Gatineau (approx. 0.36M) |

(V) Confirm against Statistics Canada and Institut de la statistique du Québec current estimates.

### The National Capital Region as launch market

Ottawa and Gatineau form a single metropolitan labour and housing market split by a provincial border, with roughly 1.5 million residents. This has three consequences that matter more than raw size:

- Two regulatory regimes apply across a thirty-minute drive. Very few operators hold credentials on both sides, which reduces effective competition for a firm that does.

- The Ontario side has higher median household income, a larger detached housing stock in the relevant price band, and materially better electricity price economics for load-shifting automation.

- The region has an exceptional concentration of the target psychographic: federal public servants, defence and security sector employees and contractors, and a technology sector cluster in Kanata. These are households that understand data sovereignty as a concept and will pay for it.

## 3.2 Market segmentation by installation depth

| Tier | Description | Typical project value (CAD) | Served by |
| --- | --- | --- | --- |
| T0 Do it yourself | Bulbs, plugs, a speaker, one camera | 150 to 800 | Amazon.ca, Costco, Best Buy, Canadian Tire, Home Depot, RONA |
| T1 Single point, installed | Smart thermostat, video doorbell, smart lock, a few circuits | 800 to 3,000 | Electricians, HVAC contractors, security franchises |
| T2 Integrated subsystem | Whole-floor lighting, multi-camera NVR, zoned HVAC, managed network | 5,000 to 15,000 | Small local integrators, low-voltage and AV shops |
| T3 Whole-house high end | Control4, Savant, Crestron, Lutron, distributed audio, motorised shading | 25,000 to 150,000 plus | Certified integrators concentrated in affluent submarkets |

Published Canadian smart home market figures are dominated by T0 and T1 product sales through retail. Using a headline market number to size a professional services business overstates the addressable opportunity by a large multiple. The plan sizes the market bottom-up instead.

### Bottom-up estimate of the professional services market

| Step | Assumption | Result |
| --- | --- | --- |
| Owner-occupied detached and semi-detached, ON and QC | Base | 5.5 to 6.5 million households |
| Share with income and dwelling profile suited to a T2 project | 8 to 12 percent | 450,000 to 780,000 households |
| Annual retrofit conversion rate for this category | 1 to 2 percent | 5,000 to 15,000 projects per year |
| Average T2 project value | CAD 8,000 | CAD 40 to 120 million per year |

This is a real but modest, highly fragmented and geographically concentrated market. It favours a focused local operator with a defensible niche and penalises attempts at rapid national scale. It also means a single firm capturing even a small share of the National Capital Region is a viable business.

## 3.3 Demand drivers

### 3.3.1 The electricity price asymmetry

|  | Ontario | Quebec |
| --- | --- | --- |
| Residential tariff structure | Time of use, plus an ultra-low overnight option, with a large peak to off-peak spread | Two-block Rate D, effectively flat through the day |
| Approximate price range | Roughly 3 c/kWh overnight on ULO to roughly 18 to 29 c/kWh on peak, depending on the plan | Roughly 7 to 11 c/kWh |
| Does automation reduce the bill? | Yes, materially. Shifting water heating, EV charging, HVAC and laundry to off-peak has a measurable payback | Barely. The spread is too small to fund the investment |
| Dominant sales argument | Economic return and load management | Comfort, security, remote control, property value, insurance-adjacent risk reduction |

(V) Confirm current tariffs with the Ontario Energy Board and Hydro-Quebec. This asymmetry must drive different sales scripts, different proposal templates and different device recommendations on each side of the river. Selling energy savings in Gatineau invites the customer to disprove the claim with their own bill.

### 3.3.2 Water damage and insurance

Water damage exceeds fire as a source of Canadian residential insurance claims by a wide margin. Several Canadian insurers offer premium credits for leak detection and automatic shut-off devices, and unlike burglar alarm credits these generally do not require a monitored connection. Leak detection therefore has the best combination of low cost, demonstrable payback and easy sale of any single device in the catalogue, and should anchor the entry-level package. (V) Confirm current credits with Intact, Desjardins, Aviva, Belairdirect and Co-operators.

### 3.3.3 Aging in place

Both provinces have aging populations and strong policy and personal preference for remaining at home. Automation for this segment emphasises automatic night lighting paths, simplified single-button control, remote check-in by adult children, temperature and water monitoring, and door status awareness. The buyer is frequently the adult child rather than the occupant, the sales cycle is longer, and price sensitivity is lower than in the general market. Competition here is thin.

### 3.3.4 Other drivers

- Heat pump conversion in both provinces, which pulls thermostat and zoning work behind it.

- Electric vehicle adoption, where a Level 2 charger installation is a natural entry point into the home and connects directly to load management.

- Baseboard heating in Quebec, where line-voltage thermostat replacement is a high-volume, low-complexity, locally manufactured product opportunity.

- Short-term rental and secondary property owners, who need access control, temperature protection against freeze-up and leak detection, and who buy per unit and repeat.

- New construction packages, where builders in the GTA and Ottawa increasingly include a base smart package and the upgrade sale to the purchaser carries the margin.

## 3.4 Direct competitors

### 3.4.1 Tier one: national subscription incumbents

| Competitor | Model | Strengths | Exploitable weaknesses |
| --- | --- | --- | --- |
| TELUS SmartHome Security | Monthly subscription, multi-year term, subsidised equipment, cloud platform, professional monitoring | National brand, bundling with mobile and internet, retail presence, financing | Customer owns nothing at end of term; cloud recording; contract lock-in; limited automation beyond the vendor catalogue; no integration with equipment the customer already owns |
| Bell Smart Home | Same structural model | Brand, bundling, installer network | Same as above |
| Vivint Canada | Long-term financed equipment plus monitoring | Aggressive direct sales, polished app | Reputation issues around door-to-door sales and contract terms (V); proprietary ecosystem; equipment effectively unusable after cancellation |
| Hilo (Hydro-Quebec subsidiary), Quebec only | Device ecosystem plus winter demand-response challenges paying customer rebates | Utility credibility, real cash incentive, local brand, Sinope partnership | Cloud dependent by design; limited scope beyond climate and a few devices; no camera or serious automation depth; the incentive requires ceding control of setpoints |

These four define the customer's mental model of "smart home security". The single most valuable sentence in the sales conversation is the one that breaks that model: if you stop paying them, your equipment stops working; if you stop paying us, nothing changes.

### 3.4.2 Tier two: custom integrators

Dealers of Control4, Savant, Crestron and Lutron, typically members of CEDIA, concentrated in affluent submarkets of Toronto and Montreal and thinly present in Ottawa and Quebec City. They deliver excellent outcomes at high prices, use proprietary programming that locks the customer to the dealer, and rarely engage below roughly twenty-five thousand dollars. They are not the initial competitor and should not be attacked head-on. In the medium term they are a referral source for projects they consider too small.

### 3.4.3 Tier three: electrical and HVAC contractors adding smart devices

This is the most direct competitor for T1 and T2 work. They already hold the licence and the customer relationship. Their consistent weaknesses are that they install but do not program, they do not design or secure the network, they do not document the system, and they do not offer structured post-installation support. The gap they leave is precisely the offer described in Chapter 4.

### 3.4.4 Tier four: retail and do-it-yourself

Unbeatable on hardware price and irrelevant on service. The correct response is never to compete on device price and always to price design, installation, programming, documentation and support as the product. A customer who has already bought devices at retail and failed to make them work reliably is a well-qualified lead, not a lost sale.

## 3.5 Competitiveness analysis

### 3.5.1 Structural forces

| Force | Intensity | Assessment |
| --- | --- | --- |
| Threat of new entrants | Low to moderate | Regulatory barriers in Quebec are high; in Ontario a low-voltage entrant can start quickly, but building the certified BOM, the security posture and the delivery method takes time |
| Bargaining power of suppliers | Moderate | Concentrated distribution, but multiple substitutable brands per function; mitigated by holding accounts with two or more distributors |
| Bargaining power of buyers | Moderate | Buyers can compare hardware prices instantly; mitigated by pricing service rather than product and by the documentation deliverable |
| Threat of substitutes | High at T0 and T1, low at T2 | Retail DIY substitutes for simple work; nothing substitutes for a designed, segmented, documented system |
| Rivalry | Moderate | Fragmented, mostly non-overlapping in positioning; the sovereignty niche is close to uncontested |

### 3.5.2 Position map

|  | Cloud dependent | Locally hosted |
| --- | --- | --- |
| Subscription required | TELUS, Bell, Vivint, Hilo | Effectively empty |
| One-time purchase | Retail DIY, most electricians adding devices | This company; a handful of custom integrators partially |

### 3.5.3 SWOT

| Strengths | Weaknesses |
| --- | --- |
| Electrical engineering background and hands-on residential wiring experience; quality management and audit discipline; trilingual capability for a bilingual market; architecture already designed and coherent; no legacy business model to protect | No Canadian trade licence yet; no installed reference base; single operator capacity; open-source stack carries a real support burden; capital constrained |

| Opportunities | Threats |
| --- | --- |
| Uncontested sovereignty niche; dense target psychographic in the National Capital Region; Quebec-manufactured device ecosystem enabling a strong buy-local story; aging-in-place segment thinly served; cross-border credential advantage in Ottawa-Gatineau | Platform obsolescence and upstream breaking changes; hardware margin compression; a national incumbent launching a local-storage tier; regulatory change; supply and tariff volatility; unbillable support creep |

## 3.6 Ideal customer profile

| Segment | Profile | Priority |
| --- | --- | --- |
| Sovereignty buyer | Detached home, technically literate, federal public service, defence supply chain, technology sector, security-cleared households, privacy-motivated, subscription-averse | Primary at launch |
| Aging in place | Homeowner over seventy or adult child purchasing on their behalf, values simplicity and remote reassurance | Secondary, high margin, low competition |
| Property protection | Second home, chalet, short-term rental, absent owner; freeze, leak and access are the drivers | Secondary, repeatable per unit |
| Comfort and convenience | Renovating homeowner, wants lighting scenes, blinds, climate; least differentiated | Tertiary, take when it arrives |

# 4. Service Offering Strategy and Perceived Value

## 4.1 Positioning statement

For homeowners who want a genuinely automated house without handing their private life to a subscription platform, the Company designs, installs, programs and documents a smart home that runs entirely on hardware the customer owns, accessible from anywhere, with no mandatory monthly fee and no third party able to see inside.

## 4.2 The sovereignty table

This table is the central sales artefact. It should appear in the proposal, on the website and on a printed leave-behind.

| Data | Leaves the property? | Where it lives |
| --- | --- | --- |
| Camera video, live | No | Customer server; viewed remotely through an encrypted point-to-point tunnel to that server |
| Camera recordings and history | No | Customer storage on customer premises |
| AI object detection and event clips | No | Processed on the customer server; no cloud inference |
| Lighting, receptacle and fan state | No | Local |
| Thermostat setpoints, temperature and energy history | No | Local |
| Automation logic and routines | No | Local |
| Occupancy, presence and household routine data | No | Local |
| Mobile push notification envelope | Minimal, see 7.6 | Notification delivery uses the mobile operating system push service; message content is configurable down to a bare alert with no detail |
| Support session, when the customer enables it | Yes, temporarily | Encrypted tunnel to the Company, enabled by the customer, expiring automatically |

Honesty on the push notification line is a competitive weapon, not a liability. Every competitor quietly relies on the same operating system push services and none disclose it. Disclosing it, explaining the mitigation and offering a self-hosted alternative demonstrates the technical seriousness the target customer is buying.

## 4.3 Service lines

| Line | Content | Revenue type |
| --- | --- | --- |
| Assessment and design | Site survey, load and circuit audit, network survey, wireless mesh planning, written design with device schedule and drawings | Fixed fee, credited against the project |
| Supply and installation | Certified hardware, line-voltage work, low-voltage cabling, rack build, PoE runs, device mounting | Project |
| Configuration and programming | Controller build, network segmentation, device onboarding, automation logic, dashboards, mobile setup | Project |
| Commissioning and handover | Resilience testing, signed acceptance record, as-built documentation, customer training session | Project |
| Year one care | Version management, remote support, one annual on-site visit, backup verification | Included in project, non-optional |
| Continuing care plan | Same as year one, renewed annually | Recurring |
| Expansion work | Additional devices, cameras, zones, rooms | Project |
| Handover-out service | Documented freeze, credential transfer, exit package for customers who do not renew | Fixed fee |

## 4.4 Pricing architecture

- Never quote hardware separately at cost-plus visible margin. Quote a delivered outcome per zone or per package, with the device schedule attached as a specification rather than a price list.

- Charge for the survey and design. A fee in the range of CAD 250 to 450, credited in full against a signed project. This filters unserious enquiries, funds the pre-sale effort, and signals professionalism.

- Include year one care in the project price and state its standalone value. This converts the single largest hidden cost of an open-source stack into a visible, paid deliverable and establishes the renewal conversation from day one.

- Publish the storage formula and the bandwidth formula. Competitors promise retention periods without doing the arithmetic. Showing the calculation in the proposal is a credibility asset with the target customer.

- Price expansion work at a published rate card so that adding a camera in year three is a two-minute conversation rather than a new negotiation.

## 4.5 How the customer perceives real value

Perceived value in this category is not created by feature lists. It is created by four specific moments, each of which must be engineered into the delivery process.

### Moment one: the unplug demonstration

During the sales visit, the technician disconnects the controller in front of the customer and switches the lights on and off from the wall. This single demonstration resolves the most common objection ("what happens when it breaks?") more effectively than any amount of explanation, and no subscription competitor can perform it, because their switches are dead without their cloud.

### Moment two: the cancel test

The customer is asked to consider what happens to their system if the Company disappears tomorrow. The answer is: nothing. The server is theirs, the credentials are in their documentation binder, the software is open source, and any competent technician can take over. This reframes the purchase from a dependency into an asset.

### Moment three: the acceptance test

At handover, the internet connection is physically disconnected and the system is demonstrated to continue working. Then the power is cut and the graceful shutdown and recovery are demonstrated. The customer signs an acceptance record listing every test passed. Almost nobody in this market does this, it takes twenty minutes, and it produces both a legal protection and a referral story.

### Moment four: the document

The customer receives a written as-built document: every device, model and location; every circuit modified; the network map and addressing; every automation described in plain language; where credentials are stored; the backup procedure; and the exact steps to hand the system to another provider. Customers routinely describe this as the moment they understood what they had bought.

## 4.6 Total cost of ownership, presented honestly

The locally hosted system is not cheaper than a subscription in year one. It is cheaper over a long horizon, and it delivers a materially larger scope. The proposal should say so plainly, because the target customer will do this arithmetic anyway and will trust a vendor who did it first.

| Five-year comparison, comparable scope | Subscription incumbent | Locally hosted system |
| --- | --- | --- |
| Up-front cost | Low, often near zero with a term commitment | CAD 8,000 to 12,000 typical mid tier |
| Monthly cost | CAD 45 to 100 depending on tier and camera count | CAD 0 in year one; CAD 30 to 65 optional thereafter |
| Five-year cash outlay | CAD 2,700 to 6,000 plus equipment charges and early termination exposure | CAD 9,400 to 14,900 |
| Ownership at year five | None; equipment typically unusable after cancellation | Full ownership of all hardware and data |
| Scope covered | Security devices and a limited device catalogue | Whole-home lighting, receptacles, fans, climate, cameras, sensors, network |
| Video retention | Limited by cloud tier, typically days | Limited only by the disks purchased, typically weeks to months |
| Price escalation risk | Yes; the provider sets the price annually | None; the customer can decline renewal and keep the system |
| Data exposure | Provider holds recordings and telemetry | None |

The honest conclusion to put in front of the customer: if you only want a doorbell camera and a monitored alarm, the subscription is cheaper and you should take it. If you want your whole house automated and you are not willing to rent it, this is the only architecture that does that.

## 4.7 Objection handling

| Objection | Response |
| --- | --- |
| "What if you go out of business?" | Everything runs on hardware you own and open-source software. Your documentation binder contains every credential and every setting. Any competent integrator can take over. Nothing is licensed to us. |
| "Open source sounds risky." | The specific projects used run on hundreds of thousands of installations worldwide and are maintained by funded organisations. The risk is not the software failing, it is the software changing, which is exactly what the care plan manages. |
| "It is more expensive than TELUS." | In year one, yes, and the scope is not comparable. Here is the five-year table, and here is what you own at the end of it. |
| "Can I really see my cameras from Mexico?" | Yes. Your phone builds an encrypted tunnel directly to your own server. The video never touches anyone else's equipment. We will demonstrate it on your phone before you pay the balance. |
| "Who else can see my cameras?" | Nobody, including us, unless you turn on the support switch. It expires automatically. |
| "What if the internet goes down?" | Everything inside the house keeps working, including recording and automation. You lose only remote access and notifications until it returns. We will demonstrate this at handover. |
| "Is this an alarm system?" | No. It alerts you, and optionally sounds a siren, but nothing is transmitted to a monitoring company and no police dispatch is arranged. That is stated in your contract. |

## 4.8 Channel strategy

| Channel | Why it works | Priority |
| --- | --- | --- |
| Home services and property preparation contractors | Already hold homeowner trust and physical access; lowest acquisition cost available | First |
| Real estate agents and stagers | Pre-sale installation and a handover package for the buyer | First |
| HVAC contractors | Natural attachment on thermostat and zoning work | Second |
| Property managers and short-term rental operators | Access control, freeze and leak protection; repeat purchase per unit | Second |
| Professional and community networks in the target psychographic | Word of mouth inside closed professional circles is the highest-converting channel for this niche | First |
| Insurance brokers | Leak detection credit gives the broker a reason to refer | Second |
| Digital advertising | Expensive and low-converting for this category at launch | Later |

Concentration risk: no single referral partner should exceed roughly thirty percent of annual revenue. Build the second and third channel before the first one becomes indispensable.

# 5. Regulatory, Licensing and Compliance Requirements

Licensing in Canada attaches to the ACTIVITY performed, not to the product sold. A single installation can simultaneously cross the electrical contracting regime, the low-voltage regime, the private security regime and the consumer protection regime. The matrix below must be read activity by activity, not project by project.

## 5.1 Activity-based licence matrix

| Activity | Ontario | Quebec |
| --- | --- | --- |
| A1. Line-voltage electrical work: relays in boxes, switches, receptacles, ceiling fans, line-voltage thermostats, new circuits, panel work | Electrical Contractor Licence (ECL) from the Electrical Safety Authority, requiring a designated Master Electrician; ESA notification and inspection per job | Regie du batiment du Quebec (RBQ) contractor licence, electrical subcategory; mandatory membership in the Corporation des maitres electriciens du Quebec (CMEQ); CCQ competency cards for on-site workers under Act R-20 |
| A2. Low-voltage: structured cabling, PoE runs, IP cameras, network, controller, programming and integration | No specific provincial installer licence; Canadian Electrical Code Class 2 rules still apply | Verify whether the RBQ classifies the work as construction; a security systems subcategory exists (V) |
| A3. Electronic security systems: cameras, intrusion sensors, local sirens | No known provincial installer licence; the Private Security and Investigative Services Act covers guards and investigators, not installers (V) | Bureau de la securite privee (BSP) agency licence under the Private Security Act, plus an agent permit for each installer. Mandatory (V) whether camera-only systems are captured |
| A4. Alarm monitoring and emergency dispatch | Licensed activity; out of scope by design | Licensed activity; out of scope by design |
| A5. Selling and contracting in the customer's home | Consumer Protection Act direct agreement rules; restrictions on unsolicited door-to-door sales of specified home services (V) | Consumer Protection Act; likely itinerant merchant permit from the Office de la protection du consommateur, with security deposit (V) |
| A6. Handling customer personal information including video | PIPEDA | Quebec Act respecting the protection of personal information in the private sector, as amended by Law 25 |
| A7. Employing people | WSIB coverage, Employment Standards Act | CNESST coverage, Act respecting labour standards, CCQ where construction work applies |

## 5.2 Ontario in detail

### 5.2.1 Electrical contracting

- The Electrical Safety Authority (ESA) administers the Ontario Electrical Safety Code and licenses electrical contractors.

- An Electrical Contractor Licence (ECL) is required for any business that carries out electrical work for others. The business must designate a Master Electrician.

- The Master Electrician designation requires a Certificate of Qualification in trade 309A, Construction and Maintenance Electrician, plus documented experience and a written examination. (V) current experience threshold and examination content.

- Trade 309A is a compulsory-certification trade in Ontario, administered by Skilled Trades Ontario. Foreign experience may be assessed through a trade equivalency assessment route, which can materially shorten the path. (V) this is a high-value question to ask directly.

- Every job requires ESA notification, a permit and inspection. Inspection outcomes become part of the property record and of the contractor's history.

### 5.2.2 Low-voltage and security

- Ontario does not maintain a provincial licence for low-voltage or security system installers, so far as is known. (V) with the Ministry of Public and Business Service Delivery.

- Class 2 wiring, cable type, penetrations, fire-stopping and plenum rating remain governed by the Ontario Electrical Safety Code and the Ontario Building Code, regardless of the absence of a licence.

- Municipal alarm by-laws apply to the property owner. Ottawa and Toronto operate alarm registration and false-alarm cost-recovery regimes. Because the design does not transmit to a monitoring centre, most of these obligations fall away, but local audible sirens are still captured by noise by-laws. (V)

## 5.3 Quebec in detail

Quebec is materially more closed than Ontario. This is a barrier at entry and a protection afterwards.

### 5.3.1 Regie du batiment du Quebec

- A contractor licence is required to carry out construction work. Licence categories and subcategories determine what may be performed; the electrical subcategory is required for line-voltage work, and a security and alarm systems subcategory exists. (V) the exact subcategory numbers, which change.

- The licence requires a designated repondant who passes examinations in business administration, project and site management, and safety. Examinations are administered in French.

- A financial guarantee or licence bond is required. (V) current amount.

- The Quebec Construction Code, Chapter V Electricity, adopts CSA C22.1 with provincial amendments.

### 5.3.2 Corporation des maitres electriciens du Quebec

Membership in the CMEQ is mandatory by statute for any contractor performing electrical work in Quebec. It is not a voluntary trade association. Membership carries annual dues, a professional liability requirement and a code of conduct. (V) current conditions and cost.

### 5.3.3 Commission de la construction du Quebec

Workers performing construction work covered by Act R-20 require a competency certificate, either compagnon or apprenti. This governs who may be on the crew, in addition to who may sign the contract. It also brings collective agreement wage rates, reporting and contribution obligations. (V) whether the specific work performed falls inside or outside the Act, which is the single most consequential scoping question in Quebec.

### 5.3.4 Bureau de la securite privee

The Private Security Act regulates the installation, repair and maintenance of electronic security systems. A company performing this activity requires an agency licence, and each individual performing it requires an agent permit. Both involve background checks, fees and renewal.

PRIORITY ACTION. Ask the BSP one precise question before any other regulatory step: does the installation of IP cameras and local sensors, WITHOUT an intrusion alarm panel and WITHOUT any monitoring connection, require an agency licence? The answer determines whether camera work can begin in Gatineau immediately or whether it is gated behind a licensing process. Obtain the answer in writing.

### 5.3.5 Ordre des ingenieurs du Quebec

The OIQ governs the title and practice of engineering in Quebec. This is a separate track from contracting. Holding an engineering credential does not authorise electrical contracting, and holding a contractor licence does not authorise use of the engineer title. A foreign engineering licence must go through the OIQ admission process to permit use of the title, and through the MIFI comparative evaluation for general recognition purposes. Neither substitutes for the trade qualification route described in 5.2.1.

## 5.4 Consumer protection: the most commonly missed requirement

Both provinces regulate contracts concluded away from the merchant's place of business, which is exactly how this business signs work. This is frequently overlooked by new home-services operators and carries real penalties.

|  | Ontario | Quebec |
| --- | --- | --- |
| Governing statute | Consumer Protection Act, 2002 | Consumer Protection Act (Loi sur la protection du consommateur) |
| Contract type | Direct agreement, when signed at the consumer's home | Contrat conclu ailleurs que dans un etablissement |
| Prescribed content | Yes; specific mandatory clauses and disclosures | Yes; stricter, with a prescribed form and mandatory cancellation notice |
| Cooling-off period | Ten days from receipt of a compliant written copy | Ten days; longer if the contract is non-compliant |
| Special permit | None known (V) | Itinerant merchant permit from the Office de la protection du consommateur, with a security deposit, likely required (V) |
| Door-to-door restrictions | Unsolicited door-to-door sales of certain specified home services are prohibited (V) whether this category is captured | Regulated; itinerant merchant rules apply |
| Consequence of non-compliance | Contract voidable, refund obligation, administrative penalties | Contract voidable, refund obligation, penalties |

Practical mitigation: have the contract templates for both provinces reviewed once by a lawyer familiar with consumer protection in home services. This is a few hours of professional fees against a class of risk that can void every contract signed.

## 5.5 Privacy and data protection

### 5.5.1 Which law applies

- Quebec: the private sector privacy Act as amended by Law 25 applies to the Company as an enterprise, including obligations on collection, consent, retention, security incident reporting, a designated person responsible for the protection of personal information, and privacy impact assessment for certain projects.

- Ontario and federal: PIPEDA applies to commercial activity.

- Both apply to the Company as a service provider, not to the household using its own cameras for personal purposes.

### 5.5.2 Where the exposure actually is

The architecture removes almost all of the exposure by design, because the Company never holds customer video. Three residual exposures remain and must be controlled:

- Support access. When the customer enables the support switch, the technician can see live video and system state. This is a collection of personal information. It requires consent, a defined purpose, a time limit, an access log and a documented policy.

- Credential custody. Passwords, network keys and remote access identities created during installation. These must never be stored in plain text, never reused across customers, and must be transferred to the customer at handover.

- Customer records. Names, addresses, floor plans, device schedules and site photographs held by the Company for quoting and service. These are ordinary personal information and require a retention schedule and secure storage.

### 5.5.3 Camera placement law

This is a design constraint that must be taught to every installer:

- Cameras may cover the customer's own property. Deliberately framing a neighbour's windows, yard or interior creates civil exposure. In Quebec, the Civil Code protects privacy and the right to one's image explicitly.

- Audio recording is treated more strictly than video in Canadian law. Disable audio capture by default and enable it only on written customer instruction with a documented rationale.

- Cameras must not be placed in bathrooms, bedrooms or any area with a reasonable expectation of privacy, and the Company should decline such requests in writing.

- For rental or shared properties, the presence of cameras and their coverage must be disclosed to occupants. Signage may be required. (V)

- Document camera fields of view in the as-built record. This is both a design deliverable and a defence.

## 5.6 Language obligations in Quebec

- The Charter of the French Language, as amended, requires contracts, invoices, catalogues, commercial documentation, signage and websites to be available in French.

- Standard-form contracts must be presented in French first; an English version may follow only after the French version has been presented. (V) the precise rule for adhesion contracts.

- Obligations scale with employee headcount, including francisation requirements above certain thresholds. (V)

- Practical consequence: every template, proposal, contract, acceptance record, as-built document and dashboard label must exist in French. Budget translation as a real line item, not an afterthought.

## 5.7 Business registration, tax and coverage

| Item | Requirement |
| --- | --- |
| Legal form | Sole proprietorship is workable at launch; incorporation is advisable before the first employee, before RBQ licensing and before significant liability exposure |
| Registration | Registraire des entreprises du Quebec for a NEQ; Ontario Business Registry for Ontario operations; extra-provincial registration to operate in both |
| GST/HST and QST | Already registered; both apply, with place-of-supply rules determining which tax applies to work performed on each side of the river |
| Workers compensation | CNESST in Quebec, WSIB in Ontario; frequently required by commercial customers even for a sole operator |
| Insurance | Commercial general liability; errors and omissions; tools and inventory; automobile business use endorsement |
| Financial guarantee | RBQ licence bond in Quebec (V); OPC security deposit if the itinerant merchant permit applies (V) |
| Municipal | Business registration or permit where required by Ottawa and Gatineau (V) |

## 5.8 Import, tariffs and supply chain compliance

- The dominant compliance reason to buy inside Canada is certification. Canadian Electrical Code rule on approved equipment requires that fixed electrical equipment bear the mark of a recognised certification body, such as cULus, cETL or CSA. European CE marking does not satisfy this. An inspector may reject the installation, and in the event of a fire an insurer has a direct argument to deny the claim.

- The dominant commercial reasons are warranty and returns handling, model continuity across a multi-year project pipeline, and lead time.

- On duty specifically: many electronic goods enter at zero duty, and the real cost of small cross-border shipments is usually courier brokerage, handling and import taxes rather than the tariff itself. However, the Canada-United States tariff environment moved significantly during 2025 and 2026 and remains volatile. (V) before fixing prices in any quotation with a validity period.

- A practical rule for the bill of materials: any device that will be installed inside an electrical box, or wired to line voltage, must be purchased from a Canadian distributor and must carry a Canadian certification mark that is physically visible on the device. No exceptions, no European stock-keeping units.

## 5.9 Compliance roadmap

| Phase | Actions | Gate cleared |
| --- | --- | --- |
| Phase 0, weeks 1 to 4 | Written answer from the BSP on camera-only scope; written answer from the ESA on the low-voltage boundary; confirm RBQ subcategory applicability; confirm OPC itinerant merchant requirement; business registration; insurance quotes | Legal ability to sell and install low-voltage work |
| Phase 1, months 2 to 6 | Obtain BSP agency licence and agent permit if required; bilingual contract templates reviewed by counsel; privacy policy and access policy published; distributor accounts opened; first reference installation completed | Ability to sell security systems in Quebec |
| Phase 2, months 6 to 18 | Trade equivalency assessment with Skilled Trades Ontario; partnership agreement with a licensed electrical contractor on each side of the river; RBQ examinations if pursuing the Quebec route | Ability to deliver full-scope projects without subcontract dependency |
| Phase 3, year 2 plus | Master Electrician and ECL in Ontario; RBQ licence and CMEQ membership in Quebec | Full independence and the highest barrier to imitation |

Do not start at Phase 3. Obtaining a trade licence before validating that customers exist consumes the capital and the time that the business needs to find product-market fit. Phase 0 and Phase 1 are sufficient to generate revenue in both provinces with a subcontracting partner for line-voltage work.

# 6. Hardware Architecture

## 6.1 Device selection rules

Five rules govern every device that enters the catalogue. They are stated first because they eliminate most of the hobbyist device ecosystem, and that elimination is the point.

- Certification. Any device installed inside an electrical box or connected to line voltage must bear a visible Canadian certification mark: cULus, cETL or CSA. European CE marking is not acceptable. This is a Canadian Electrical Code requirement, not a preference.

- Radio compliance. Any device containing a radio must carry an ISED certification number. This applies to battery-powered sensors too, which are otherwise outside the electrical certification regime.

- Local control. The device must be fully controllable without an internet connection and without a manufacturer account. A device that requires cloud authentication to change state is disqualified regardless of price or features.

- Canadian procurement. The device must be purchasable from a Canadian distributor or Canadian retail. No European stock-keeping units, no direct-from-manufacturer imports for anything that goes in a wall.

- Mechanical fallback. Lighting and receptacle control must continue to function from the physical switch when the controller is offline.

Every certification statement in this chapter is a starting point and must be confirmed against the specific stock-keeping unit at the moment of purchase, because manufacturers ship differently certified variants of the same model name for different markets. Verify the mark on the physical device, not on the web page.

## 6.2 Protocol strategy

| Protocol | Role in the design | Notes |
| --- | --- | --- |
| Zigbee | Primary protocol for switches, dimmers, thermostats, sensors, plugs | Mature, local, mesh, very wide device support; mains-powered devices act as routers and strengthen the mesh; managed through an open translation layer rather than a proprietary hub |
| Z-Wave | Secondary, where Zigbee coverage or device availability is weak | Lower interference risk because it avoids the 2.4 GHz band; smaller catalogue; useful for long-range door sensors in large homes |
| Matter and Thread | Selective, forward-looking | Good local control model; the ecosystem is still consolidating, so adopt device by device rather than as a platform commitment |
| Wi-Fi | Restricted use | Acceptable for a small number of relay modules on a dedicated isolated network segment; never for battery sensors; every Wi-Fi device is a node on a network that must be secured |
| Power over Ethernet | Cameras, coordinators, access points, touch panels | Single cable for data and power, centralised backup, no local power supply to fail |
| Wired low-voltage | Contacts, sirens where practical | Highest reliability, no batteries, no radio; preferred wherever the wall is open |

Design rule: one Zigbee network per property, one coordinator, positioned centrally and away from the rack. Do not mix multiple competing hubs. Every additional radio ecosystem multiplies the support burden without adding capability.

## 6.3 Lighting

### 6.3.1 Switch first, bulb second

The default is always to control the circuit at the switch, not to replace the bulb. Smart bulbs fail the mechanical fallback rule: if someone switches the wall off, the bulb is unreachable. Smart bulbs are used only where colour or colour-temperature control is specifically wanted, or where no neutral and no room for a module exist.

### 6.3.2 The no-neutral problem

A large share of the Ontario and Quebec housing stock built before the 1980s has no neutral conductor in the switch box. This is the single most common technical obstacle in Canadian retrofit work and must be identified during the survey, not on installation day.

| Situation | Solution | Notes |
| --- | --- | --- |
| Neutral present in switch box | Standard smart switch or dimmer, or micromodule behind the existing switch | Preferred; widest device choice |
| No neutral, standard box | No-neutral capable dimmer designed for the North American market | Lutron Caseta is the reference solution for this case; verify LED fixture compatibility |
| No neutral, no room in box | Module at the fixture canopy, or bulb-level control | Requires access to the fixture |
| Multi-way (three-way and four-way) circuits | Companion or remote devices from the same family, or a canopy module with wireless wall remotes | Never mix vendors across a multi-way group |
| Aluminium branch wiring | Pigtail with approved connectors, by a licensed electrician | Common in certain 1960s and 1970s Canadian housing; an insurance issue in its own right and a natural upsell |

### 6.3.3 LED dimming compatibility

Most dimming complaints in modern retrofit work are LED compatibility problems, not device faults. Symptoms are flicker, buzzing, a narrow usable dimming range, and failure to turn off completely. Controls:

- Confirm the dimmer's published compatibility list against the actual fixture and lamp models before quoting.

- Prefer to supply the lamps as part of the project, so the compatibility pairing is controlled rather than inherited.

- Set minimum and maximum dim levels during commissioning for each circuit; this is a programming task, not a hardware fix.

- Respect the derating rule for multiple dimmers ganged in one box, which reduces the permitted load per device.

### 6.3.4 Lighting device catalogue

| Function | Recommended | Why |
| --- | --- | --- |
| Dimmers and switches, no-neutral capable | Lutron Caseta | The most reliable no-neutral solution in the North American market; strong CSA and cULus position; local control through a bridge with a documented local interface (V) on the model with local access |
| General smart switches and dimmers | Leviton Decora Smart; Inovelli Blue Series 2-1 | Leviton has universal Canadian distribution and unambiguous certification; Inovelli is cETL listed, Zigbee, extremely well supported by open-source controllers, and offers scene control and an LED status bar |
| Quebec-manufactured switches, dimmers and controllers | Sinope Technologies | CSA certified, Zigbee, made in Quebec, available through Quebec electrical distribution; strong buy-local argument |
| Colour and tunable-white lamps | Philips Hue; Sengled; IKEA | Broad Canadian retail availability; usable without the manufacturer cloud; reserve for accent and effect lighting only |
| Fixture-level modules | Canopy-mount modules certified for Canada | For no-neutral, no-space situations (V) current certified options |

## 6.4 Relays and micromodules

Micromodules preserve the customer's existing switch plates and the existing physical circuit, which matters aesthetically and for the mechanical fallback rule. The constraint is certification.

| Status | Devices | Action |
| --- | --- | --- |
| Approved | Shelly, North American stock-keeping units only, with a visible UL or ETL mark; Inovelli modules; DIN-rail relays certified for Canada | Verify the mark on the physical unit at receiving; reject unmarked stock |
| Rejected | Sonoff ZBMINIL2; Aqara relay modules; any European stock-keeping unit; any device with CE marking only | Removed from the catalogue. These appear in early draft designs and must not be carried forward |

The rejected devices are cheaper, popular in the hobbyist community, and technically capable. Installing them in a Canadian electrical box exposes the customer to an inspection failure and an insurance denial, and exposes the installer to the resulting liability. This is a category where the correct decision costs money and must be priced in.

## 6.5 Receptacles and plug loads

| Application | Approach | Devices |
| --- | --- | --- |
| Permanently controlled outlet | In-wall smart receptacle | Leviton Decora Smart outlet; Enbrighten in-wall outlet (V) certification per SKU |
| Flexible or temporary control | Plug-in smart plug | Sinope smart plug (CSA, Quebec); Canadian-retail Zigbee plugs; these also extend the Zigbee mesh |
| Energy metering | Metering plug or metering module | Useful for load-shifting proposals in Ontario; near-useless as a savings argument in Quebec |
| High-draw appliances | Dedicated load controller by a licensed electrician | Water heater, EV charger, baseboard circuits; Sinope load controllers are the standard Quebec answer |

## 6.6 Fans

SAFETY RULE, NON-NEGOTIABLE: never install a lighting dimmer on a ceiling fan motor. It is a fire and motor-failure hazard and a frequent installer error. Ceiling fans require a fan-rated speed control or a simple on-off relay.

| Application | Approach |
| --- | --- |
| Ceiling fan speed control | Fan-rated smart control from Lutron, Leviton or Inovelli; verify the device is explicitly rated for fan motor loads |
| Ceiling fan with integrated light | Separate control paths for fan and light; canopy modules where the box has a single switch leg |
| Bathroom exhaust fan | Smart switch plus humidity sensor automation, or a fan with an integrated condensation sensor; automate a run-on timer after occupancy ends |
| Whole-house mechanical ventilation | Do not automate code-required ventilation interlocks. In newer Quebec construction the bathroom fan may be tied to the heat recovery ventilator; verify before touching the circuit |

## 6.7 Climate control

### 6.7.1 The Quebec case: line-voltage heating

The Quebec housing stock is dominated by electric baseboard heating, which requires line-voltage thermostats rated for the connected load. This is a large, high-volume, locally manufactured, technically simple opportunity and it should be treated as a core product line rather than an accessory.

| Load | Device family | Notes |
| --- | --- | --- |
| Baseboard heaters up to approximately 3,000 W | Sinope line-voltage Zigbee thermostat; Stelpro Zigbee thermostat | Both Quebec manufacturers; CSA certified; local Zigbee control; no cloud required |
| Baseboard heaters to approximately 4,000 W | Higher-rated model in the same family | Confirm the load calculation per circuit before specifying |
| Electric floor heating | Floor-heating thermostat with floor sensor from the same families | Verify sensor compatibility with the existing installed floor sensor |
| Convectors and fan heaters | Manufacturer-matched controls | Some units require a specific control type |

### 6.7.2 The Ontario case: low-voltage HVAC

| System | Device | Notes |
| --- | --- | --- |
| Forced air furnace and central air conditioning | Ecobee (Toronto); Honeywell Home T6 Pro Z-Wave | Both offer local control paths; Ecobee through Matter or the HomeKit local interface, avoiding dependence on the manufacturer cloud |
| Heat pump, ducted or ductless | Manufacturer thermostat plus infrared or CN105 serial interface for mini-splits | Mini-split control through a local ESP-based interface gives full offline control; this is a differentiator against every cloud competitor |
| Zoned systems | Zone panel with per-zone thermostats | Do not modify zone panel safety interlocks |
| Hydronic and boiler systems | Thermostat only | Never touch boiler internal or combustion controls; out of scope |

## 6.8 Sensors

| Type | Purpose | Selection notes |
| --- | --- | --- |
| Water leak | Highest-value sensor in the Canadian market; insurance credit is achievable without monitoring | Sinope leak sensors with a Sinope motorised shut-off valve gives detection and automatic response, all local, Quebec-made |
| Door and window contact | Intrusion alerting, garage door status, HVAC interlock | Wired contacts where walls are open; Zigbee battery contacts for retrofit; verify ISED certification |
| Motion and occupancy | Lighting automation and intrusion alerting | Passive infrared for reliability; millimetre-wave presence sensors for rooms where people sit still, at higher cost and higher configuration effort |
| Temperature and humidity | Comfort automation, freeze protection, bathroom fan control | Freeze alerting for secondary properties is a strong standalone product |
| Air quality | Comfort and ventilation control | Carbon dioxide and particulate sensors; explicitly not a gas safety device and must be labelled as such |
| Energy metering | Load-shifting proposals, per-circuit visibility | Panel-level current transformers require a licensed electrician |

Sensor naming discipline: no sensor in this system may be labelled, described or configured in a way that implies life-safety function. An air quality sensor is not a gas detector. A temperature sensor is not a fire detector. This applies to dashboard labels and automation names as well as to documentation.

## 6.9 Local audible alerting

| Device | Placement | Configuration |
| --- | --- | --- |
| Indoor siren, Zigbee or wired | Central hallway, upper floor | Distinct tone pattern; never the fire evacuation temporal-three pattern; automatic cut-off within the municipal by-law limit |
| Outdoor siren and strobe | Front elevation, out of reach | Weather-rated enclosure; tamper contact; cut-off timer mandatory |
| Chime and voice announcement | Interior speakers | Preferred for non-emergency alerts such as door opened or leak detected |

## 6.10 Cameras

### 6.10.1 Selection criteria

- Native RTSP and ONVIF support, so the camera is not tied to a manufacturer application.

- Power over Ethernet in preference to Wi-Fi for every fixed exterior camera.

- A usable secondary stream at low resolution for AI detection, alongside the main stream for recording.

- Metal housing, IP66 or better, and a rated operating range covering Canadian winter, which means down to at least minus thirty degrees Celsius. Plastic consumer cameras fail in the Ottawa and Gatineau climate.

- No mandatory cloud account for basic operation.

### 6.10.2 Vendor lineage and the government-adjacent customer

For an ordinary residential customer, camera manufacturer origin is irrelevant. For the primary target segment in the National Capital Region, which includes federal employees and defence supply chain households, it is a purchase criterion. Several widely sold North American camera brands are rebadged from Chinese manufacturers whose position in Canada has been the subject of government action. (V) the current status before publishing any brand claim.

Practical policy: maintain at least one camera line with a clearly non-restricted supply chain, disclose lineage openly in the proposal when asked, and never present a rebadged unit as a domestic product. Transparency here converts a risk into a trust-building moment.

### 6.10.3 Camera catalogue

| Tier | Option | Position |
| --- | --- | --- |
| Integrated and simple | Ubiquiti UniFi Protect cameras with a UniFi recording appliance | Recording is fully local on customer-owned hardware; excellent build quality; strong Canadian availability; a closed but genuinely local ecosystem, appropriate for a customer who values reliability over openness |
| Open and flexible | Reolink PoE line | Good price and image quality; RTSP and ONVIF support varies by model and firmware, so (V) model by model before committing to a purchase batch |
| Premium and supply-chain sensitive | Axis Communications; Hanwha Vision | Sold through Canadian security distribution; the correct answer for the security-cleared or defence-adjacent household; higher cost, justified by the customer segment |
| Excluded | Any unit with an unclear or restricted supply chain lineage | Kept out of the catalogue rather than managed case by case |

## 6.11 Compute and recording platform

| Component | Specification | Rationale |
| --- | --- | --- |
| Controller and recorder host | Refurbished business-class micro desktop: Dell OptiPlex Micro, Lenovo ThinkCentre Tiny, or HP EliteDesk Mini, with a modern Intel processor including integrated graphics, 16 to 32 GB RAM | Business-grade build, three to five year lifecycle, quiet, low power, purchased inside Canada from Canadian refurbishers, and the integrated graphics provide hardware video decoding and AI inference without an add-on accelerator |
| AI acceleration | Integrated Intel graphics as the default path | Removes dependence on a discrete USB accelerator whose supply has been unreliable; a discrete accelerator becomes optional at the largest tier only |
| System disk | NVMe solid-state drive, 250 to 500 GB | Operating system, database and configuration only |
| Video storage | Surveillance-rated mechanical drives: WD Purple or Seagate SkyHawk | Designed for continuous write workloads; consumer solid-state drives are consumed by continuous video recording and fail without warning |
| Bulk storage at larger tiers | Network attached storage with redundancy, or additional internal drives in a mirrored set | Redundancy protects against drive failure; it is not a backup |

## 6.12 Storage sizing

Publish this calculation in every proposal. Competitors quote retention periods without performing it.

STORAGE FORMULA:  Terabytes = (bitrate in Mbps / 8) x 86,400 seconds x number of cameras x retention days / 1,000,000.  A simplified working rule: one camera at 8 Mbps consumes approximately 86 GB per day.

| Configuration | Continuous recording | Storage for 14 days | Storage for 30 days |
| --- | --- | --- | --- |
| 4 cameras at 4 Mbps (2K, efficient codec) | 172 GB per day | approx. 2.4 TB | approx. 5.2 TB |
| 6 cameras at 8 Mbps (4K) | 516 GB per day | approx. 7.2 TB | approx. 15.5 TB |
| 10 cameras at 8 Mbps (4K) | 860 GB per day | approx. 12.0 TB | approx. 25.8 TB |
| 16 cameras at 6 Mbps (mixed) | 1,036 GB per day | approx. 14.5 TB | approx. 31.1 TB |

Hybrid retention policy, which reduces these figures substantially and should be the default offer: continuous recording retained for a short window such as seven days, and AI-detected events involving people or vehicles retained for a long window such as sixty days. This typically reduces total storage by half to two thirds while preserving everything the customer actually wants to find later.

## 6.13 Network hardware

| Component | Requirement | Options |
| --- | --- | --- |
| Router and firewall | VLAN support, inter-VLAN firewall rules with stateful inspection, IDS optional | UniFi gateway appliance; TP-Link Omada gateway; open-source firewall on Canadian-purchased hardware |
| Switching | Managed Layer 2, 802.1Q VLAN tagging, PoE+ budget sized to the camera count with headroom | UniFi PoE switches; TP-Link Omada; MikroTik for cost-sensitive builds |
| Wireless | Multiple SSIDs mapped to separate VLANs, WPA3 where supported, band steering control | UniFi access points; Omada access points |
| Cabling | Cat6 or Cat6A, riser-rated (CMR) for in-wall, plenum-rated (CMP) where required by the building code | Purchase in Canada; verify the jacket rating marking on the cable itself |
| Rack or enclosure | Ventilated, lockable, with cable management and a labelled patch panel | Wall-mount rack for typical residential installations |

## 6.14 Power and continuity

| Element | Specification | Notes |
| --- | --- | --- |
| Uninterruptible power supply | Line-interactive or online, pure sine wave output, USB data port, sized for the full rack load | Pure sine wave matters: modern power supplies with active power factor correction can misbehave on simulated sine wave output |
| Monitoring | Network UPS Tools integration into the controller | Enables state visibility, low-battery alerts and orderly shutdown |
| Runtime target, standard | Fifteen to thirty minutes | Sufficient for orderly shutdown through a normal outage |
| Runtime target, security-priority | Two to four hours for the camera and recorder subsystem only | Addresses the deliberate power interruption scenario, which is the one that matters for a security system; achieved with a dedicated UPS for the PoE switch and recorder, or a lithium iron phosphate backup |
| Internet continuity | Optional cellular failover | Without it, a power cut that takes down the service provider equipment also takes down remote notification |

## 6.15 Radio coordinators

| Element | Requirement |
| --- | --- |
| Zigbee coordinator | A single coordinator per property, placed at the geometric centre of the device population, away from the rack and away from the router. A network-attached coordinator powered over Ethernet is preferred because it can be placed anywhere a cable reaches. Where only USB coordinators are available in Canada, extend with a shielded USB cable and keep it clear of USB 3 ports and solid-state drives, which are strong 2.4 GHz noise sources |
| Z-Wave controller | Only where a specific device requires it; do not run a Z-Wave network without a reason |
| Channel planning | Select a Zigbee channel that does not overlap the Wi-Fi channels in use on site; survey before commissioning, not after complaints |
| Mesh construction | Mains-powered devices act as routers. Install and pair mains devices first, then battery devices, so that battery devices join through a mature mesh |

## 6.16 Canadian supply chain

| Channel | Suppliers | Use |
| --- | --- | --- |
| Security and low-voltage distribution | ADI Global Distribution Canada; Aartech Canada | Cameras, sensors, cabling, racks, Z-Wave and Zigbee catalogue |
| Electrical distribution, national | Nedco; Westburne; Guillevin International | Switches, dimmers, receptacles, boxes, wire, thermostats |
| Electrical distribution, Quebec | Franklin Empire; Lumen; Guillevin | Sinope and Stelpro lines, French-language support, local counter pickup |
| Manufacturer or dealer direct | Sinope Technologies; Stelpro; Lutron dealer channel; Ubiquiti Canadian resellers | Core product lines; dealer pricing requires an account and, for some, a licence |
| Information technology retail | Canada Computers; Memory Express; Newegg.ca | Network gear, drives, uninterruptible power supplies |
| Refurbished business hardware | Canadian refurbishers of off-lease business desktops | Controller and recorder hosts at a fraction of new cost |

Open distributor accounts BEFORE the first paid project, not after. Distributor pricing versus consumer retail pricing is frequently the difference between a viable and an unviable gross margin, and several accounts require a business number, proof of insurance and in some cases a licence.

## 6.17 Excluded device register

Maintain this as a living document. A device enters the register when it fails a selection rule, with the reason recorded, so that the same evaluation is not repeated.

| Device or category | Reason for exclusion |
| --- | --- |
| Sonoff ZBMINIL2, Aqara relay modules, any CE-only relay | No confirmed Canadian certification for fixed installation |
| Any device requiring a manufacturer cloud account to operate | Violates local control rule and the core value proposition |
| Consumer plastic-body outdoor cameras | Cold-weather failure; no PoE; weak RTSP support |
| Cameras with unclear or restricted supply chain lineage | Unacceptable to the primary customer segment |
| Lighting dimmers applied to fan motors | Fire and equipment hazard |
| Consumer solid-state drives for continuous video recording | Write endurance failure without warning |
| European stock-keeping units of otherwise approved brands | Wrong certification and wrong voltage or frequency assumptions |

# 7. Software Architecture, Cybersecurity and Data Protection

The customer is buying a promise that their data does not leave their house. That promise is only as good as the security posture of the equipment left behind. A locally hosted system that is poorly secured is worse than a cloud service, because the customer believes they are protected and no vendor security team is watching. Security is therefore not an add-on in this business; it is the product.

## 7.1 Software stack

| Layer | Component | Licence | Role |
| --- | --- | --- | --- |
| Host | Home Assistant Operating System, or a minimal Linux host running containers | Apache 2.0 | Managed base with supervised updates and add-on isolation |
| Automation core | Home Assistant Core | Apache 2.0 | Device state, automation logic, user interface, mobile application backend |
| Messaging | Eclipse Mosquitto | EPL / EDL | Local publish and subscribe bus between devices and the core |
| Zigbee | Zigbee2MQTT, or the built-in Zigbee integration | GPL-3.0 | Protocol translation, decoupling devices from proprietary hubs |
| Z-Wave | Z-Wave JS UI | MIT | Z-Wave network management where used |
| Video | Frigate NVR | MIT | Recording, hardware-accelerated decoding, local object detection |
| Streaming | go2rtc | MIT | Low-latency stream handling and restreaming to clients |
| Custom firmware | ESPHome | Open source | Fully local control of infrared, serial and custom sensor nodes |
| Remote access | WireGuard, with a mesh overlay control plane | GPL-2.0 (WireGuard) | Encrypted point-to-point access with no inbound port exposure |
| Reverse proxy | Traefik or NGINX | MIT / BSD | TLS termination inside the property |
| Identity | Authelia or Authentik | Apache 2.0 / MIT | Optional single sign-on and multi-factor enforcement at larger installations |
| Credential vault | Vaultwarden | AGPL-3.0 | Per-customer credential custody, handed to the customer at closeout |
| Metrics and history | InfluxDB or VictoriaMetrics, with Grafana | MIT / Apache 2.0 / AGPL-3.0 (Grafana) | Long-term energy and environment history |
| Availability monitoring | Uptime Kuma | MIT | Local health checks and alerting |
| Backup | Restic or BorgBackup, with rclone | BSD-2 / BSD-3 / MIT | Encrypted configuration backup, on-site and optional customer-controlled off-site |
| Power management | Network UPS Tools | GPL-2.0 | UPS state and orderly shutdown |
| Edge security | CrowdSec or equivalent | MIT | Behavioural blocking at the perimeter |
| Fleet management | Ansible | GPL-3.0 | Reproducible deployment across customers; the Company's internal tool |

## 7.2 Open-source licence obligations

This is a commercial and legal matter, not a philosophical one, and it is routinely ignored by integrators. Delivering software on hardware sold to a customer is distribution in the legal sense of these licences.

- Unmodified distribution of Apache 2.0, MIT and BSD components requires preserving copyright and licence notices. Deliver a licence appendix with every installation.

- GPL and AGPL components that the Company modifies trigger an obligation to provide the corresponding source to the recipient. Zigbee2MQTT, ESPHome, Network UPS Tools and Grafana fall in this family.

- Policy: do not fork. Configure, template and extend through supported mechanisms rather than patching upstream code. Where a patch is genuinely necessary, publish it in a public repository and reference that repository in the customer documentation. This converts a compliance burden into a two-line answer.

- Trademark discipline. Project names may be used descriptively but not in a way that implies endorsement, certification or partnership. The Company sells its own service, built on named open-source projects.

- No warranty pass-through. Every one of these licences disclaims warranty. The contract must state that the Company warrants its installation and configuration work, and does not and cannot warrant the underlying software.

## 7.3 Deployment standard

- Configuration as code. Every installation is generated from a versioned template held in the Company's private repository, with a per-customer variables file. Nothing is hand-built.

- Reproducibility. A destroyed controller must be rebuildable from the template plus the customer backup in under four hours, with no tribal knowledge.

- Version pinning. Container images and add-on versions are pinned. Automatic updates are disabled in production. Updates are applied deliberately, after testing on the Company bench.

- Naming convention. Devices, areas, entities and automations follow a documented naming standard so that any technician can read any installation.

- No unique snowflakes. Customer-specific behaviour lives in the variables file and in automation definitions, never in undocumented manual edits.

## 7.4 Network segmentation

Segmentation is what prevents a compromised camera or relay from becoming access to the household's personal devices. The design uses a single physical network with enforced logical separation, which is both more secure and cheaper than parallel cabling.

| Segment | Contents | Internet access | Notes |
| --- | --- | --- | --- |
| Trusted | Family phones, laptops, tablets, printers | Full | Normal household use |
| IoT | Wi-Fi relays, thermostats, plugs, touch panels | Restricted: time synchronisation and specific update endpoints only | No path to Trusted |
| Camera | IP cameras only | None | Cameras accept connections from the recorder and originate nothing |
| Controller | Home Assistant and recording host | Restricted: updates and package repositories only | Reaches IoT and Camera; reachable from Trusted and from the remote access tunnel |
| Management | Gateway, switches, access points, out-of-band interfaces | None, except deliberate firmware retrieval | Accessible only from an administrative device or an authorised support session |
| Guest | Visitors | Full internet, isolated from everything else | Client isolation enabled |

### 7.4.1 Firewall policy matrix

| From \ To | Trusted | IoT | Camera | Controller | Management |
| --- | --- | --- | --- | --- | --- |
| Trusted | - | Allow specific | Deny | Allow specific | Deny by default |
| IoT | Deny | - | Deny | Allow specific | Deny |
| Camera | Deny | Deny | - | Deny (the recorder initiates) | Deny |
| Controller | Deny | Allow | Allow | - | Deny by default |
| Guest | Deny | Deny | Deny | Deny | Deny |

The camera rule is directional and stateful. The recorder opens the connection to the camera; the camera can never open a connection to anything. A rule written simply as "block the camera VLAN" is not equivalent and will either break recording or leave a return path open. Document the rules as directional in the as-built record.

### 7.4.2 Practical consequences of the isolated camera segment

- Time synchronisation. Cameras with the wrong clock produce recordings with the wrong timestamps, which destroys their evidentiary value. Provide a local time server inside the segment rather than opening an outbound exception.

- Firmware updates. Camera firmware becomes a manual, scheduled maintenance task performed by the technician. This is an argument for the care plan, not a defect.

- Manufacturer applications will not work. Tell the customer this explicitly during design, because it is the most common surprise at handover. Everything is viewed through the single system interface instead.

## 7.5 Identity and access control

| Control | Standard |
| --- | --- |
| Accounts | Named accounts per household member. No shared administrative account. No account named "admin" |
| Multi-factor authentication | Mandatory on the controller, the network management interface and the storage appliance for every account with administrative rights |
| Default credentials | Every device credential changed at commissioning, including cameras, switches, access points and the storage appliance. Verified by checklist, not by memory |
| Password policy | Unique, generated, minimum sixteen characters, stored only in the credential vault |
| Company access | No standing account on any customer system. Support access is created for a session and expires. See 7.7 |
| Credential handover | At project closeout, the customer receives the vault and an administrative recovery procedure. The Company retains no copy |
| Least privilege | Household members receive user-level accounts; administrative rights only where genuinely required |
| Physical | Rack or equipment cabinet lockable; console access restricted |

## 7.6 Remote access, and one honest disclosure

### 7.6.1 Architecture

Remote access uses a WireGuard-based encrypted overlay. The customer's phone establishes a direct encrypted tunnel to their own controller. No ports are opened on the router, no traffic is relayed through a third-party media server, and video is served by the customer's own hardware to the customer's own device.

- No inbound exposure. The router presents no open ports to the internet, which removes an entire class of attack.

- Direct peer connection. Where the network permits, the tunnel is established directly between phone and home. Where symmetric network address translation prevents this, the connection falls back to an encrypted relay, which cannot decrypt the traffic passing through it.

- Control-plane options. The commercial overlay service is faster to deploy and requires a business subscription for commercial use (V). A self-hosted open-source control plane removes the third party entirely at the cost of additional operational responsibility. Offer both, default to the self-hosted option for customers whose motivation is sovereignty.

- Key custody. Keys belong to the customer. The Company holds no persistent key material for any customer installation.

### 7.6.2 The push notification disclosure

This must be stated plainly to the customer, in writing, because it is the one place where the "nothing leaves the house" claim requires a footnote.

Mobile push notifications on Android and on iOS are delivered through the operating system vendor's push infrastructure. This is an architectural property of the mobile platforms and cannot be avoided by any application, from any vendor, without the phone maintaining a permanent foreground connection, which is impractical on iOS and costly to battery life on Android.

Mitigations applied by default:

- Notification content is configured to the minimum: an event category and a time, with no image, no location detail and no camera name. What passes through the push service is the equivalent of "motion detected".

- The image or clip is fetched only when the customer opens the application, at which point it travels over the encrypted tunnel from their own server.

- Customers who want zero third-party involvement can choose local-only alerting: in-home audible and visual alerts, plus a self-hosted notification channel reachable when the phone is on the home network or on the tunnel.

- The choice is presented as a documented option in the design, not decided silently by the installer.

Every competitor relies on the same push infrastructure and none of them disclose it. Disclosing it, explaining the mitigation, and offering an alternative is the single most persuasive demonstration of technical honesty available in the sales conversation with this customer segment.

## 7.7 Support access model

This is a signature feature of the offer and it must be implemented precisely.

- The Company holds no standing access to any customer system.

- The customer enables support through a clearly labelled control in their own dashboard, or by a single tap in the mobile application.

- Enabling it creates a time-limited authorisation, defaulting to two hours, implemented as an explicit automation with a timer that revokes access on expiry, on reboot, and on manual cancellation.

- Every enable, connection, and expiry event is logged locally on the customer system in a log the customer can read.

- The customer receives a notification when a support session begins and when it ends.

- Support sessions are additionally recorded in the Company work-order record, with the technician name, the reason and the actions taken, which is the evidence required under privacy law.

- At project closeout and on request at any time, the customer can permanently remove Company access with a documented procedure.

## 7.8 Update and version management

| Class | Policy | Cadence |
| --- | --- | --- |
| Security patches, host operating system | Applied promptly after bench validation | Within days of release |
| Controller and add-on versions | Pinned; never automatic; validated on the Company bench installation first | Monthly review, quarterly application unless a security issue forces it |
| Device firmware | Only when it fixes a defect affecting the installation or closes a vulnerability; never for its own sake | As required |
| Breaking-change screening | Release notes reviewed before every update; installations with affected integrations flagged in advance | Every release cycle |
| Rollback | A full snapshot is taken before every update; documented rollback procedure | Every update |
| Customers not on a care plan | Version frozen at handover; written notice that updates are their responsibility; documented procedure provided | At non-renewal |

This single policy is the difference between a sustainable business and unpaid support work. An unmanaged open-source stack degrades, and the customer attributes the degradation to the installer regardless of whose decision it was.

## 7.9 Backup and recovery

| Item | Standard |
| --- | --- |
| What is backed up | Controller configuration, automations, dashboards, device database, network configuration exports, credential vault. Video is not backed up; it is protected by drive redundancy only, and the customer is told so explicitly |
| Encryption | Backups encrypted at rest with a key held by the customer |
| On-site copy | Second local target, separate physical device |
| Off-site copy | Optional and customer-controlled: their own cloud storage account, or an encrypted external drive rotated by the customer. The Company does not host customer backups |
| Redundancy is not backup | Mirrored or parity drive sets protect against drive failure, not against deletion, corruption or ransomware. Filesystem snapshots on the storage appliance mitigate the second category |
| Restore testing | A restore is performed and verified at least once per year as part of the care plan, and evidenced in the service record |
| Recovery objective | Full rebuild from template plus backup in under four hours |

## 7.10 Hardening checklist

Applied at every installation and signed off at commissioning:

- Host firewall enabled, default deny inbound.

- No password-based remote shell access; key-based only, or shell disabled entirely.

- All default credentials changed and recorded in the vault.

- Multi-factor authentication enforced on all administrative accounts.

- Universal Plug and Play disabled on the gateway.

- No port forwarding rules. Confirmed by external scan at commissioning.

- Remote management of the gateway from the internet disabled.

- Cameras: default account removed, unused services disabled, cloud and peer-to-peer features disabled, audio disabled unless explicitly requested, firmware current at handover.

- Wireless: WPA3 or WPA2-AES, no WPS, unique keys per segment, management network not exposed over wireless.

- Guest network isolated with client isolation enabled.

- Unused switch ports administratively disabled.

- Logging enabled on gateway and controller with a defined retention period.

- External vulnerability scan of the customer's public address performed and the clean result attached to the acceptance record.

## 7.11 Data protection policy

| Obligation | Implementation |
| --- | --- |
| Purpose limitation | Customer information collected only for quotation, design, installation and support. Purposes stated in the contract |
| Consent | Explicit written consent for site photographs and for support access. Support access consent is renewed by the customer at every session through the enable control |
| Minimisation | The Company holds no customer video, no live feeds, no recordings, and no ongoing telemetry |
| Retention | Quotation and design records retained for a defined period, then destroyed. Site photographs deleted after project closeout unless the customer authorises retention |
| Security safeguards | Encrypted Company workstations and backups; credential vault; no customer credentials in email, chat, spreadsheets or notes |
| Person responsible | A named individual designated as responsible for the protection of personal information, published in the privacy policy |
| Transparency | Published privacy policy in French and English describing exactly what is collected, why, for how long and who can access it |
| Incident response | Documented procedure, see 7.12 |
| Customer rights | Documented procedure for access, correction and deletion requests, with a response service level |
| Cross-border | No customer personal information stored outside Canada. Company tooling selected accordingly |

## 7.12 Security incident response

- Detect. Local monitoring alerts, customer report, or upstream vulnerability disclosure affecting a deployed component.

- Contain. Isolate the affected segment or disable the affected service. The segmentation design makes containment a firewall change rather than a rebuild.

- Assess. Determine whether personal information was accessed. Because the Company holds no customer video, most incidents affect availability rather than confidentiality, which materially reduces the reporting burden.

- Notify. Where personal information is involved, notify the customer without delay, and notify the regulator where the legal threshold for a serious incident is met. Quebec requires a register of confidentiality incidents regardless of severity. (V) current thresholds and register requirements.

- Remediate and record. Patch, rotate credentials, verify, and record the incident in the register with the timeline and the actions taken.

- Review. Assess whether the same exposure exists at other installations, and apply the fix across the fleet through the configuration templates.

## 7.13 Logging and auditability

- Controller, gateway and storage logs retained locally for a defined period, sized so that a month-old question can still be answered.

- Support session log kept in a form the customer can read without technical assistance.

- Configuration changes tracked in version control on the Company side, so that "what changed and when" is always answerable.

- Work orders record every visit, every remote session, the reason, and the changes made. This is both good practice and the evidence base required if a privacy complaint is ever made.

# 8. Packages, Sizing and Network Requirements

## 8.1 Package definitions

|  | S — Essential | M — Standard | L — Premium | XL — Estate / light commercial |
| --- | --- | --- | --- | --- |
| Target property | Condo, townhouse, small bungalow | Detached family home | Large detached, multi-level, secondary building | Large property, multiple buildings, small business premises |
| Automated devices | Up to 25 | 25 to 60 | 60 to 120 | 120 to 250+ |
| Cameras | 2 to 3 | 4 to 6 | 7 to 12 | 13 to 24 |
| Camera resolution | 2K | 2K to 4K mixed | 4K | 4K |
| Continuous retention | 7 days | 7 to 10 days | 14 days | 14 to 30 days |
| Event retention | 30 days | 60 days | 60 to 90 days | 90 days |
| Controller | Refurbished micro desktop, 16 GB RAM | Refurbished micro desktop, 16 to 32 GB RAM | Small form factor desktop, 32 GB RAM, discrete storage bay | Rack server or split controller and recorder hosts |
| Storage | 250 GB NVMe + 4 TB surveillance HDD | 500 GB NVMe + 8 TB surveillance HDD | 500 GB NVMe + 2 x 12 TB mirrored | NAS with redundancy, 32 TB usable or more |
| Network | Gateway + 8-port PoE+ switch + 1 access point | Gateway + 16-port PoE+ switch + 2 access points | Gateway + 24-port PoE+ switch + 3 to 4 access points | Gateway + multiple switches + wireless design + fibre uplinks between buildings |
| UPS | 1000 VA | 1500 VA | 1500 VA + dedicated camera UPS | Rack UPS + dedicated camera UPS |
| VLANs | 4 | 5 | 6 | 6 or more, plus per-tenant separation |
| Installation effort | 2 to 3 days | 4 to 6 days | 7 to 12 days | 12 days or more |
| Indicative project value (CAD) | 3,500 to 6,000 | 7,000 to 13,000 | 15,000 to 30,000 | 30,000 and up |
| Care plan (from year two) | 30 per month | 45 per month | 65 per month | Quoted |

Package prices are indicative and must be rebuilt from actual distributor pricing once accounts are open. Hardware is roughly 40 to 50 percent of project value at every tier; the remainder is labour, programming, commissioning and documentation.

## 8.2 Network design

### 8.2.1 Addressing plan

A single addressing standard is applied at every installation so that any technician can navigate any site without reading the documentation first.

| VLAN | Name | Subnet convention | DHCP | Present in |
| --- | --- | --- | --- | --- |
| 10 | Trusted | 10.x.10.0/24 | Yes | All tiers |
| 20 | IoT | 10.x.20.0/24 | Yes, with reservations | All tiers |
| 30 | Camera | 10.x.30.0/24 | Static reservations only | All tiers |
| 40 | Controller | 10.x.40.0/24 | Static | All tiers |
| 50 | Management | 10.x.50.0/24 | Static | M and above; folded into Controller at tier S |
| 60 | Guest | 10.x.60.0/24 | Yes, isolated | M and above |

The second octet is assigned per customer from a Company register, so that two sites never collide when a technician is connected to both through the support overlay. This is a small discipline that prevents a real and confusing class of failure.

### 8.2.2 Power over Ethernet budget

| Device | Typical draw | Class |
| --- | --- | --- |
| Fixed dome or bullet camera, 2K | 4 to 8 W | PoE (802.3af) |
| Fixed camera, 4K with infrared illumination at night | 8 to 13 W | PoE+ (802.3at) |
| Camera with heater for winter operation | 15 to 25 W | PoE+ or PoE++ |
| Wireless access point | 10 to 20 W | PoE+ |
| Network-attached Zigbee coordinator | 2 to 4 W | PoE |
| Touch panel | 10 to 15 W | PoE+ |

Sizing rule: total the worst-case draw of all powered devices, including night-time infrared and heater load, and specify a switch with at least 40 percent budget headroom. Undersized PoE budget produces intermittent camera dropouts at night that are extremely difficult to diagnose after the fact, because they only appear in cold and dark conditions.

### 8.2.3 Cabling standard

- Cat6 minimum for all camera and access point runs; Cat6A where runs approach one hundred metres or where future upgrade is anticipated.

- Riser-rated (CMR) jacket for in-wall vertical runs; plenum-rated (CMP) where required by the building code. Verify the rating printed on the cable jacket at receiving.

- Every run terminated on a labelled patch panel. No cable terminated directly into a switch without a panel except in tier S.

- Every drop labelled at both ends with a code that matches the as-built drawing.

- Exterior runs in conduit or rated for direct exposure; drip loops at every penetration; fire-stopping at every floor and fire separation crossing.

## 8.3 Internet bandwidth requirements

### 8.3.1 The structural advantage

A locally hosted system consumes almost no internet bandwidth at rest. Cameras record to a disk in the house, not to a server on the internet. Continuous cloud recording, by contrast, uploads every second of every camera stream, every day, forever. For a Canadian household on a metered or capped plan this is a real, quantifiable monthly cost that the customer is currently paying and does not see.

| Scenario | Sustained upload | Monthly data uploaded |
| --- | --- | --- |
| Locally hosted, idle (no one watching) | Effectively zero | Under 1 GB |
| Locally hosted, one hour of remote viewing per day at 2 Mbps | Intermittent | Approximately 27 GB |
| Cloud service, 4 cameras, continuous upload at 2 Mbps each | 8 Mbps continuous | Approximately 2,600 GB |

(V) the exact behaviour of any specific cloud competitor, since some upload only on event. The point stands directionally and is worth stating in the proposal.

### 8.3.2 Minimum internet service required

| Package | Minimum download | Minimum upload | Reason |
| --- | --- | --- | --- |
| S | 25 Mbps | 5 Mbps | One remote sub-stream at a time plus normal household use |
| M | 50 Mbps | 10 Mbps | Two concurrent remote streams, occasional main-stream viewing |
| L | 100 Mbps | 25 Mbps | Multiple concurrent remote viewers, main-stream review, larger household |
| XL | 300 Mbps | 50 Mbps symmetric preferred | Multiple simultaneous users, multi-building, remote review of archived footage |

- Upload is the binding constraint, not download. Canadian cable service is strongly asymmetric; fibre service is generally symmetric. Check the customer's actual upload speed during the site survey, not the advertised plan speed.

- Always stream the sub-stream by default for remote viewing and switch to the main stream only on demand. A remote viewer pulling four 4K main streams will saturate almost any residential upload link and will be perceived as a system fault.

- Carrier-grade address translation on some providers prevents direct tunnel establishment and forces relayed connections, which adds latency. Identify this during the survey; a static address or a different provider tier may be needed at tier L and above. (V) per provider.

- Cellular failover is recommended at tier L and above so that a wired outage does not remove remote access and notification.

## 8.4 Wireless mesh planning

| Consideration | Rule |
| --- | --- |
| Coordinator placement | Geometric centre of the device population, at least two metres from the rack, the gateway and any USB 3 device or solid-state drive |
| Channel selection | Choose a Zigbee channel that does not overlap the Wi-Fi channels in use at the site; survey both bands before commissioning |
| Router density | At least one mains-powered routing device per floor and per major wing; typically no more than ten to twelve metres between routing devices through residential construction |
| Pairing order | Commission all mains-powered devices first, then battery devices, so battery devices join through an established mesh and select good parents |
| Construction penalties | Concrete floors, foil-backed insulation, radiant floor heating, metal studs and stone all attenuate heavily; add routing devices rather than increasing transmit power |
| Device count per coordinator | Comfortably to roughly 100 to 150 devices with a healthy mesh; beyond this, split into a second network or move to a wired approach |
| Battery devices | Never used where a wired contact is practical; battery replacement across sixty sensors is a support burden the customer will resent |

## 8.5 Electrical and physical requirements at the rack

| Item | Requirement |
| --- | --- |
| Circuit | Dedicated 15 A branch circuit for the equipment rack at tier M and above |
| Load | Tier S approximately 60 to 120 W; tier M approximately 120 to 220 W; tier L approximately 220 to 400 W; tier XL higher, calculated |
| Ventilation | Passive ventilation minimum; forced ventilation where the enclosure is closed or in a confined space |
| Location | Conditioned space, not an unheated garage or an uninsulated attic; away from water lines and the laundry |
| Security | Lockable enclosure. An equipment rack that an intruder can switch off in ten seconds undermines the entire camera system |
| Grounding and bonding | Rack bonded per code; surge protection at the service entrance recommended and quoted separately |
| Access | Serviceable clearance in front and behind; no equipment placed where a technician cannot reach it without moving furniture |

# 9. Delivery Operations

## 9.1 Delivery process

| Stage | Activities | Output |
| --- | --- | --- |
| 1. Qualification | Telephone or video call; scope, property type, motivation, budget range; disqualify poor fits early | Decision to survey |
| 2. Site survey (paid) | Circuit and panel audit, neutral verification at every intended switch location, network and upload speed test, wireless survey, camera sightline and mounting assessment, structural and cable path assessment, photograph record | Survey report |
| 3. Design | Device schedule, network plan and addressing, camera plan with fields of view, storage and bandwidth calculation, automation narrative, package selection and price | Signed design and quotation |
| 4. Bench staging | Controller build from template, firmware flashing, device pre-pairing, container deployment, configuration generation, bench test of every device before it leaves the workshop | Pre-configured system |
| 5. Installation | Line-voltage work by the licensed contractor or partner, low-voltage cabling, rack build, camera mounting and aiming, device installation | Physically complete system |
| 6. Commissioning | Network verification, hardening checklist, mesh health check, automation tuning, dashboard build, mobile device setup for each household member, resilience testing | Working, tuned system |
| 7. Handover | Customer training session, as-built documentation package, credential vault transfer, signed acceptance record, care plan activation | Closed project |

Stage 4 is the difference between a professional operation and a hobbyist one. Every device is paired, tested and configured on the bench before it goes to site. Field time is the most expensive time in this business and must be spent on physical work, not on troubleshooting a device that could have been proven in the workshop.

## 9.2 Site survey checklist

- Electrical panel: capacity, available spaces, panel brand and age, aluminium branch wiring, knob-and-tube, grounding condition.

- Every intended switch location: neutral present, box depth and volume, gang count, multi-way configuration, existing device type.

- Lighting: fixture and lamp types per circuit, dimmable or not, existing dimmers.

- Heating: type, per-room load in watts, existing thermostat type and wiring, panel circuits.

- Ventilation: bathroom fan type and control, presence of a heat recovery ventilator and any code-required interlock.

- Network: existing gateway and switch, cable runs available, structured wiring panel, internet provider, measured download and upload speed, presence of carrier-grade address translation.

- Camera positions: sightlines, mounting surface, cable path, power availability, sun angle and glare, and the neighbour privacy assessment for every proposed field of view.

- Rack location: space, circuit, ventilation, security, service access.

- Construction: wall and floor materials, insulation type, finished ceilings, accessible attic or basement, fire separations.

- Household: number of occupants, devices per person, mobile platforms, technical comfort level, accessibility needs, pets (which affect motion sensor and camera configuration).

- Photographs of every panel, every intended device location and every camera sightline.

## 9.3 Acceptance test protocol

Performed in front of the customer at handover, and recorded on a signed acceptance record. This is a sales asset, a quality control and a legal protection simultaneously.

| Test | Expected result |
| --- | --- |
| Disconnect the controller; operate every automated light and fan from the wall | All loads operate normally from physical switches |
| Disconnect the internet service | Automation, recording, local control and in-home alerting continue; remote access unavailable as designed |
| Cut mains power to the rack | UPS carries the load; orderly shutdown occurs at the configured threshold; cameras and recorder remain live for the specified security runtime |
| Restore power | System returns to full service without manual intervention within the stated time |
| External port scan of the customer public address | No open inbound ports |
| Remote access from a mobile network | Live and recorded video and full control confirmed on each household member's device |
| Camera segment isolation | Cameras cannot reach the internet or the trusted segment; recorder can reach cameras |
| Support switch | Access enables, is logged, notifies the customer, and expires automatically at the configured time |
| Backup and restore | A backup is taken and a restore verified |
| Detection accuracy | Person and vehicle detection demonstrated on each camera with the tuned configuration |
| Notification path | Each household member receives the configured alert on their own device |

## 9.4 Documentation package

Delivered in print and as a file, in the customer's language of choice, and updated at every service visit:

- System overview and scope statement, including the exclusions from Chapter 2.

- Device schedule: every device, model, serial number, location, circuit and radio protocol.

- Electrical record: circuits modified, permits, inspection references.

- Network diagram, addressing table, VLAN and firewall rule summary in plain language.

- Camera plan with fields of view and mounting details.

- Storage and retention configuration with the calculation used.

- Automation catalogue: every automation described in plain language, with its trigger and its effect.

- Credential and recovery procedure, and the location of the credential vault.

- Backup procedure and restore procedure.

- Update policy and version state at handover.

- Support access procedure and the customer's revocation procedure.

- Continuity section: exactly how to hand this system to another provider, including what to give them and what they will need to know.

- Signed acceptance record with the completed test protocol.

## 9.5 Support plan structure

|  | Year one (included) | Care plan (renewal) | No plan |
| --- | --- | --- | --- |
| Remote support | Included, response within one business day | Included, response within one business day | Billed at the published hourly rate |
| Version management | Included | Included | Frozen at handover; customer responsibility |
| Security patching | Included | Included | Customer responsibility |
| On-site visit | One annual visit included | One annual visit included | Billed |
| Backup verification and restore test | Included | Included | Not performed |
| Battery replacement programme | Included at the annual visit | Included at the annual visit | Customer responsibility |
| Camera firmware maintenance | Included | Included | Customer responsibility |
| Priority scheduling | Standard | Priority over non-plan customers | Lowest |
| Documentation updates | Included | Included | Not provided |

The care plan is not a service contract with a response guarantee in the alarm-industry sense, and must not be described as one. It is a maintenance and support subscription. Keep this language consistent with the scope statement in Chapter 2.

## 9.6 Quality management

A documented quality system is unusual in this trade at this company size and is therefore a differentiator, particularly with the institutional-adjacent customer segment that recognises what an audit trail means.

- Documented procedures for survey, design, staging, installation, commissioning and handover, each with its checklist.

- Every checklist retained as a record against the project number.

- Non-conformance record: what went wrong, root cause, corrective action, and whether the fix applies to other installations.

- Supplier evaluation: delivery reliability, defect rate, return handling, per distributor and per manufacturer.

- Annual internal audit against the documented procedures.

- Customer satisfaction measurement at handover and at the annual visit.

- Continuous improvement review each quarter, feeding changes back into the templates and checklists.

# 10. Personnel and Competency Requirements

## 10.1 Staffing by phase

| Phase | Period | Operational roles | Administrative roles | Capacity |
| --- | --- | --- | --- | --- |
| Phase 0 Validation | Months 0 to 6 | Founder performing survey, design, staging, low-voltage installation, commissioning and support. Licensed electrical contractor engaged per project as a subcontractor | Bookkeeping outsourced. Legal review purchased once. Translation purchased per document | 6 to 10 projects |
| Phase 1 First hire | Months 6 to 18 | Founder as technical director and estimator. One field technician for cabling, mounting, device installation and staging. Same electrical subcontract arrangement | Part-time administrator for scheduling, purchasing, invoicing and records. Bookkeeping still outsourced | 18 to 30 projects per year |
| Phase 2 Consolidation | Year 2 to 3 | Founder. Two field technicians. One remote support and systems technician. Licensed electrician either employed or on a formalised partnership agreement | Full-time administrator. Bookkeeping outsourced. Compliance responsibilities formally assigned | 40 to 70 projects per year plus a growing care-plan base |

The first hire should be a field technician, not a salesperson. In this business the constraint at launch is delivery hours, not lead flow, and the founder is the only credible salesperson for a technical proposition until there is a reference base.

## 10.2 Role definitions

### 10.2.1 Founder and Technical Director

Accountable for design, estimating, customer relationship, regulatory compliance, supplier relationships and the technical standard. Performs the site survey and presents the design personally at Phase 0 and Phase 1, because the sale depends on visible technical authority. Owns the configuration templates and the quality system.

### 10.2.2 Field Technician

Performs low-voltage cabling, camera mounting and aiming, rack build, device installation, bench staging and first-line commissioning. Works under the direction of the licensed electrician for any line-voltage task. Does not design and does not quote.

### 10.2.3 Licensed Electrician

Performs or directly supervises all line-voltage work; holds or is covered by the applicable provincial licence; pulls permits; signs off on electrical work. Engaged as a subcontractor at Phase 0 and Phase 1, on a written agreement that states clearly who owns the customer relationship, who is responsible for warranty on electrical work, and how the two firms handle a callback.

### 10.2.4 Support and Systems Technician

Handles remote support tickets, version testing on the bench, patch management, fleet configuration, backup verification and the care-plan annual visits. This role becomes necessary as soon as the care-plan base passes roughly forty installations, and it is the role that makes recurring revenue profitable rather than merely present.

### 10.2.5 Administrator

Scheduling, purchasing and receiving, supplier accounts, invoicing and collections, permit paperwork, records retention, customer correspondence in both languages, and maintenance of the project record files. This role is chronically underestimated by technical founders and is usually the highest-return early hire after the first technician.

## 10.3 Competency matrix

Levels: 1 = awareness, 2 = can perform with supervision, 3 = independent, 4 = can design and teach.

| Knowledge domain | Founder | Field Tech | Support Tech | Admin |
| --- | --- | --- | --- | --- |
| Canadian Electrical Code, load calculation, circuit identification | 4 | 2 | 1 | - |
| Residential wiring practice: neutrals, multi-way, aluminium, panel work | 4 | 2 | 1 | - |
| LED and dimmer compatibility, fan-rated controls | 4 | 3 | 2 | - |
| Structured cabling, termination, testing, cable ratings | 3 | 4 | 1 | - |
| TCP/IP, VLANs, 802.1Q, routing, DHCP, DNS | 4 | 2 | 4 | - |
| Firewall policy design and stateful rule construction | 4 | 1 | 4 | - |
| Wireless radio frequency planning, channel selection, site survey | 3 | 3 | 3 | - |
| Power over Ethernet standards and budget calculation | 4 | 3 | 3 | - |
| Linux command line, containers, services, logs, storage | 3 | 2 | 4 | - |
| Home Assistant: entities, automations, templating, dashboards | 4 | 2 | 4 | - |
| Zigbee and Z-Wave mesh behaviour and troubleshooting | 4 | 3 | 4 | - |
| Video: codecs, bitrate, lens and field of view selection, infrared, storage arithmetic | 4 | 3 | 3 | - |
| NVR configuration and object detection tuning | 3 | 2 | 4 | - |
| Cybersecurity hardening, credential management, incident response | 4 | 2 | 4 | 1 |
| Privacy law obligations and camera placement law | 4 | 2 | 2 | 3 |
| Working at heights, ladder and roof safety, electrical safety | 4 | 4 | 1 | 1 |
| Estimating, quoting and margin control | 4 | 1 | 1 | 2 |
| Customer training and expectation setting | 4 | 3 | 3 | 2 |
| Documentation and record keeping | 4 | 3 | 3 | 4 |
| French and English written and spoken | 3 | 3 | 2 | 4 |
| Purchasing, receiving and supplier account management | 3 | 2 | 1 | 4 |

## 10.4 Mandatory training and certification

| Item | Who | Jurisdiction | Notes |
| --- | --- | --- | --- |
| WHMIS | All field personnel | Both | Refresh periodically |
| Working at Heights, approved provider | All field personnel | Ontario | Required for construction work at height in Ontario (V) applicability to this scope |
| Construction health and safety course, 30 hours | All field personnel | Quebec | Prerequisite for a CCQ competency certificate (V) |
| CCQ competency certificate | Field personnel on covered construction work | Quebec | Required where Act R-20 applies |
| BSP agent permit | Anyone installing security systems | Quebec | Subject to the scope answer from the BSP |
| Standard first aid and CPR | All field personnel | Both | Good practice; sometimes a customer requirement |
| Networking fundamentals certification | Founder, Support Tech | Both | Externally recognised proof of competence for the institutional-adjacent customer |
| Vendor network certification | Founder, Support Tech | Both | Improves distributor standing and supports warranty claims |
| Electrical trade qualification | Founder, long term | Both | The Phase 3 objective; see Chapter 5 |

## 10.5 Knowledge management

The single largest operational risk in a small technical firm is that the knowledge lives in one head. Countermeasures, to be built from the first project rather than retrofitted:

- Configuration templates in version control, so that every installation is generated rather than remembered.

- An internal runbook covering every recurring task: pairing a device, replacing a camera, restoring a controller, onboarding a household member, handling a version upgrade.

- A defect and resolution log searchable by symptom, built from every support ticket.

- A standing bench installation in the workshop that mirrors the production template, used for testing every update before it touches a customer.

- A written onboarding path for a new technician, with a target of productive independence on low-voltage work within six weeks.

# 11. Financial Outline

All figures in this chapter are planning estimates for structuring purposes. They must be rebuilt from real distributor pricing, real insurance quotations and real licence fees before use in any financing application.

## 11.1 Start-up costs

| Category | Indicative range (CAD) | Notes |
| --- | --- | --- |
| Business registration, legal form, extra-provincial registration | 500 to 1,500 | Both provinces |
| Legal review of bilingual contract templates and consumer protection compliance | 2,000 to 5,000 | The highest-return professional spend in the plan |
| Licences and permits, Phase 0 and Phase 1 | 1,000 to 6,000 | Depends heavily on the BSP scope answer |
| Insurance, first year | 2,500 to 6,000 | General liability plus errors and omissions |
| Tools and test equipment | 4,000 to 9,000 | Cable tester and certifier, network analyser, multimeter, clamp meter, circuit tracer, thermal camera, drills, fish tape, ladders, fall protection |
| Demonstration and bench system | 5,000 to 9,000 | A fully working reference installation. The most effective sales asset available; treat as marketing spend, not overhead |
| Initial inventory and consumables | 4,000 to 8,000 | Cable, connectors, boxes, common devices, spares |
| Information technology and business systems | 3,000 to 6,000 | Laptop, quoting and customer records system, configuration repository, credential management |
| Branding, website, bilingual print material | 3,000 to 7,000 | Must exist in French |
| Translation | 1,500 to 3,500 | Contracts, proposals, documentation templates, dashboards |
| Working capital | 15,000 to 30,000 | Six months of operating cost plus project float; customers pay in stages, suppliers do not |
| Total | 42,000 to 91,000 | Excludes vehicle if already owned |

## 11.2 Unit economics, package M

| Line | Amount (CAD) | Note |
| --- | --- | --- |
| Project price | 10,000 | Mid point of the M band |
| Hardware cost at distributor pricing | 3,600 | Approximately 36 percent of price |
| Subcontracted electrical work | 1,400 | Phase 0 and Phase 1 model |
| Direct labour, own | 1,800 | Five days at loaded cost |
| Permits, consumables, travel | 400 |  |
| Gross margin | 2,800 | 28 percent |
| Same project once electrical is performed in house | 3,700 to 4,000 | 37 to 40 percent, which is the Phase 3 prize |

## 11.3 Recurring revenue model

| Care-plan base | Average monthly fee | Annual recurring revenue | Meaning |
| --- | --- | --- | --- |
| 25 customers | 45 | 13,500 | Covers software, tooling and part of insurance |
| 75 customers | 45 | 40,500 | Covers the support technician in part |
| 150 customers | 45 | 81,000 | Covers a full support role and produces surplus |
| 300 customers | 45 | 162,000 | The recurring base becomes the primary enterprise value |

Recurring revenue at scale carries a low marginal cost because the work is fleet-wide rather than per-customer: one version test protects every installation. This is the argument for standardising ruthlessly on templates from the first project.

## 11.4 Break-even

| Item | Monthly (CAD) |
| --- | --- |
| Founder draw | 4,500 |
| Administrator, part time from Phase 1 | 1,800 |
| Insurance | 400 |
| Vehicle, fuel, tools | 900 |
| Software, systems, communications | 350 |
| Premises and storage | 700 |
| Professional fees, licences amortised | 500 |
| Marketing | 600 |
| Total fixed monthly cost | 9,750 |
| Contribution per M project | 2,800 |
| Break-even volume | Approximately 3.5 projects per month, or 42 per year |
| Effect of 100 care plans | Reduces break-even by approximately 1.6 projects per month |

The realistic Phase 1 capacity of a founder plus one technician is roughly 18 to 30 projects per year. The plan therefore does not break even at Phase 1 on projects alone with a full founder draw, which is the central financial fact of the first eighteen months. Three levers close the gap: a reduced founder draw during validation, a mix that includes higher-value L projects, and the accumulation of care plans. Model all three before committing.

# 12. Risk Register

| Risk | Impact | Likelihood | Mitigation |
| --- | --- | --- | --- |
| BSP scope answer requires a licence before any camera work in Quebec | High | Medium | Ask first, before spending on Quebec marketing; operate Ontario-side while the licence is pursued |
| Uncertified device installed and rejected at inspection | High | Low if the register is enforced | Certification mark verified physically at receiving; excluded-device register maintained |
| Consumer protection non-compliance voids contracts | High | Medium | Legal review of templates before the first signature; itinerant merchant permit confirmed |
| Upstream breaking change disrupts many installations at once | Medium | High | Version pinning, bench testing, staged rollout, care plan funding the work |
| Unbilled support consumes capacity | High | High | Mandatory year one, structured renewal, freeze procedure for non-renewals, published hourly rate |
| Single referral channel dependency | Medium | Medium | Cap any single partner at roughly 30 percent of revenue; build three channels in parallel |
| Founder is the single point of knowledge | High | High | Templates, runbooks, defect log, documented onboarding from project one |
| Hardware margin compression | Medium | High | Price service, not product; distributor accounts; recurring revenue |
| Supply and tariff volatility | Medium | High | Two distributors per category; quotation validity periods; Canadian-sourced BOM |
| Privacy incident during a support session | High | Low | No standing access, time-limited sessions, logging, documented policy, errors and omissions cover |
| Customer misunderstands the system as a monitored alarm | High | Medium | Scope statement in the contract, restated at handover, recorded on the acceptance record |
| Cash flow strain from supplier terms versus customer stages | Medium | High | Deposit on signature, progress payment at delivery of hardware to site, balance at acceptance |
| A national incumbent launches a local-storage tier | Medium | Low | The moat is the whole-home scope and the openness, not the storage location alone; deepen the documentation and continuity advantages |

# 13. Implementation Roadmap

| Horizon | Objectives |
| --- | --- |
| First 30 days | Written scope answers from the BSP and the ESA. Business and tax registration confirmed for both provinces. Insurance quotations obtained. Distributor account applications submitted. Demonstration system specified and ordered. Contract templates drafted for legal review |
| Days 31 to 90 | Demonstration system built and running as the bench reference. Configuration templates and Ansible deployment built and versioned. Bilingual contract templates reviewed and issued. Privacy policy and access policy published. Website and leave-behind material produced in both languages. Electrical subcontract partner agreed in writing on each side of the river. First two projects delivered, at cost if necessary, in exchange for reference and photography rights |
| Days 91 to 180 | BSP licence obtained if required. Five to eight projects delivered. Runbook and defect log populated from real work. Care plan structure activated. First referral channel partnership formalised. Pricing rebuilt from actual distributor costs. Trade equivalency assessment application submitted to Skilled Trades Ontario |
| Days 181 to 365 | First field technician hired and productive. Fifteen to twenty-five projects delivered cumulatively. Care-plan base above twenty. Second and third referral channels active. Quality system documented and first internal audit performed. Decision taken on the Quebec RBQ and CMEQ route with real data behind it |
| Year 2 | Second technician and part-time administrator. Care-plan base above sixty. Support and systems role established. Electrical licensing route actively pursued. Light commercial and property-manager segment opened |

# 14. Verification Checklist

Every item marked (V) in this plan, consolidated. Each requires a primary source, a written answer where possible, and a date. Treat this as a controlled document.

| # | Question | Source |
| --- | --- | --- |
| 1 | Does installing IP cameras and local sensors without an intrusion panel and without monitoring require an agency licence? | Bureau de la securite privee |
| 2 | Agent permit requirements, cost, background check and timeline | Bureau de la securite privee |
| 3 | Exact boundary between unlicensed low-voltage work and work requiring an ECL, with concrete examples | Electrical Safety Authority |
| 4 | ECL and Master Electrician requirements, current experience threshold and fees | Electrical Safety Authority |
| 5 | Trade equivalency assessment route for foreign experience toward trade 309A | Skilled Trades Ontario |
| 6 | Applicable RBQ licence subcategories for electrical and for security systems, and current bond amount | Regie du batiment du Quebec |
| 7 | CMEQ membership conditions, dues and professional requirements | CMEQ |
| 8 | Whether this scope of work falls inside Act R-20 and requires CCQ competency certificates | Commission de la construction du Quebec |
| 9 | Whether an itinerant merchant permit and security deposit are required for contracts signed in the customer home | Office de la protection du consommateur |
| 10 | Direct agreement content requirements and the current door-to-door sales restrictions | Ontario Ministry of Public and Business Service Delivery |
| 11 | Law 25 obligations for an enterprise of this size, incident register requirements and reporting thresholds | Commission d'acces a l'information du Quebec |
| 12 | Language obligations by employee headcount, and the adhesion contract rule | Office quebecois de la langue francaise |
| 13 | Municipal noise by-law limits for audible sirens, and alarm registration requirements | Ottawa, Gatineau, Montreal |
| 14 | Current residential tariff structures and rates | Ontario Energy Board, Hydro-Quebec |
| 15 | Insurance premium credits for leak detection and for security devices, and any recognised installer programme | Intact, Desjardins, Aviva, Belairdirect, Co-operators |
| 16 | General liability and errors and omissions premiums for this activity and scope | Commercial insurance broker |
| 17 | Current tariff and customs treatment for the device categories in the bill of materials | Canada Border Services Agency, customs broker |
| 18 | Canadian certification status, per stock-keeping unit, for every device in the catalogue | Manufacturer and distributor, plus physical inspection |
| 19 | RTSP and ONVIF support per camera model and firmware version | Manufacturer, plus bench testing |
| 20 | Current supply chain lineage and restriction status of every camera brand carried | Public record, distributor |
| 21 | Commercial licensing terms for the remote access overlay control plane | Vendor |
| 22 | Distributor pricing and account requirements, per supplier | ADI, Aartech, Nedco, Guillevin, Franklin Empire, Lumen |
| 23 | Working at Heights and construction safety training applicability to this scope | Ontario Ministry of Labour, ASP Construction |
| 24 | Carrier-grade address translation practice and static address availability, per internet provider | Bell, Rogers, Videotron, regional providers |

# Appendix A. Working Formulas

Video storage. Terabytes = (bitrate in Mbps divided by 8) multiplied by 86,400 seconds, multiplied by the number of cameras, multiplied by retention days, divided by 1,000,000. Working rule: one camera at 8 Mbps consumes approximately 86 GB per day.

PoE budget. Sum the worst-case draw of every powered device, including night-time infrared illumination and any heater load, then add 40 percent headroom. Specify a switch whose total PoE budget exceeds that figure.

Remote viewing bandwidth. Required upload in Mbps equals the sub-stream bitrate multiplied by the number of concurrent remote viewers, plus a margin for the household's other upstream traffic. Default to sub-stream for remote viewing; escalate to main stream only on demand.

Zigbee mesh density. At least one mains-powered routing device per floor and per major wing, with no more than roughly ten to twelve metres between routing devices through residential construction; add routing devices rather than increasing transmit power.

Break-even volume. Total fixed monthly cost divided by the contribution margin per project, adjusted downward by recurring care-plan revenue divided by the same contribution figure.

# Appendix B. Glossary

| Term | Meaning |
| --- | --- |
| CEC | Canadian Electrical Code, CSA C22.1, adopted with amendments by each province |
| ECL | Electrical Contractor Licence, issued in Ontario by the Electrical Safety Authority |
| ESA | Electrical Safety Authority, Ontario |
| RBQ | Regie du batiment du Quebec, the Quebec construction contractor regulator |
| CMEQ | Corporation des maitres electriciens du Quebec, mandatory for electrical contractors in Quebec |
| CCQ | Commission de la construction du Quebec, workforce competency and labour relations in construction |
| BSP | Bureau de la securite privee, the Quebec private security regulator |
| OPC | Office de la protection du consommateur, the Quebec consumer protection regulator |
| ISED | Innovation, Science and Economic Development Canada, which certifies radio equipment sold in Canada |
| cULus, cETL, CSA | Certification marks recognised for electrical equipment in Canada |
| PoE | Power over Ethernet: power and data delivered on a single network cable |
| VLAN | Virtual Local Area Network: logical separation of traffic on shared physical infrastructure |
| RTSP and ONVIF | Open standards for video streaming and camera interoperability, which prevent vendor lock-in |
| NVR | Network Video Recorder |
| Sub-stream | A second, lower-resolution stream from the same camera, used for detection and remote viewing |
| Mechanical fallback | The design property that loads continue to operate from the physical switch when the controller is offline |